==SCH.DOC=================Tango-Schematic================Version 1.50==

INTRODUCTION

Tango-Schematic Version 1.50 contains the enhancements and  bug  fixes,

described below.

Information  in  this document file is to be used in  conjunction  with

that  contained in the Tango-Schematic Reference Manual and the  Tango-

Schematic Version 1.50 Release Notes.

We are continually working to improve our libraries.  Consequently, pin

positions,  names,  etc., may change for some components  between  this

release and those to follow.  Please take this into account as you  use

the  library  components,  realizing that  some  editing  might  become

necessary  when  the  next  release is  received.   Also,  we  strongly

recommend that you do not add your own components to any ACCEL supplied

library,  but  instead, place any such components in a  custom  library

which will not be overwritten by subsequent library updates.

The  following items have been added/changed since the Reference Manual

and Release notes were published:

INSTALLATION INSTRUCTIONS

ACCEL  has standardized on an automatic installation program,  INSTALL,

which  uses compressed files on the distribution diskettes.  The Tango-

Schematic  Reference  Manual contains complete instructions  for  using

this  utility.   The  minimum  free disk  space  required  for  a  full

installation of this release is 3.0 MB.

NEW FEATURES AND ENHANCEMENTS

The following features have been added in this release:

OUTPUT  REPORTS FILE VIEWER - Another output option has been  added  to

the  Output Reports destination group: the "View" option sends the

output  directly to the Viewer.  From the viewer, the entire  file

can be inspected, or output to a file or port.

DRIVERS  -  Several  graphics drivers and a printer  driver  have  been

added:

VESA800.DRV     The  VESA800 graphics  driver  supports  VESA

graphics at 800x600 resolution.

VESA1024.DRV    The  VESA1024 graphics driver  supports  VESA

graphics at 1024x768 resolution.

VESA1280.DRV    The  VESA1200 graphics driver  supports  VESA

graphics in 1280x1024 resolution.

VESA8G.DRV      The  VESA8G  graphics  driver  supports  VESA

graphics in 800x600 resolution, for VESA implementations with

Red and Green switched (GRB instead of RGB).

VESA10G.DRV     The  VESA10G graphics  driver  supports  VESA

graphics  in  1024x768  resolution, for VESA  implementations

with Red and Green switched (GRB instead of RGB).

VESA12G.DRV     The  VESA12G graphics  driver  supports  VESA

graphics  in  1280x1024 resolution, for VESA  implementations

with Red and Green switched (GRB instead of RGB).

See VESA.DOC for more information.

BJ180.DRV A Bubble Jet printer driver is now available.  This

driver  supports A and B sized Black/White output at  180x180

dpi for Cannon Bubble Jet printers.

KNOWN PROBLEMS

1.   MACROS - Under extreme low-memory conditions, you may receive  two

out-of-memory  error  messages while  recording  a  macro.   After

acknowledging the first, the program may appear to be hung but  it

is  actually  waiting for another acknowledgment: hit  <Enter>  to

return to normal processing.

2.   POLYGONS  -  Complicated polygons are those with many simultaneous

vertical edges; e.g., a polygon that looks like a horizontal  saw,

with  all  of  its  points at the same y-coordinate.   Complicated

polygons  outline  correctly, but do not  fill  correctly  on  the

display.   Also,  there  may  not  be  enough  memory  to   output

complicated  polygons,  depending on the  size,  scaling  factors,

available memory, and driver used (the Postscript drivers are  not

affected).  If you run out of memory while outputting, you can try

purging  the Undo information (File DOS will do this),  break  the

polygon into two or more smaller polygons, or rotate the image  to

change  the vertical edges into horizontal ones.  Also,  extremely

narrow polygons may not be displayed at some zoom levels.

ADDITIONAL CONSIDERATIONS

1.   COMPILE-FILE CHANGES - The compile file remembers the current part

names.   Whenever  the  Post  Forward Annotate  or  Back  Annotate

commands  are used to change the sheet, the compile file  must  be

updated  with the changes, or they will not be used in other  post

commands  because  the current part names have not  changed.   For

example: When compiling for the first time, it is common to  group

parts.   Forward  Annotate uses the compile file to  generate  and

display new names--but the compile file still remembers the  "old"

names  as  being  current.  If you try to  Back  Annotate  without

recompiling  first,  Schematic  will  complain  about   mismatches

between the was-is file and the current compile-file settings.

2.   EDIT PIN LENGTH - Pin length can no longer be changed while in the

sheet editor--only in the Library Editor.

3.   POSTSCRIPT  CARTRIDGES  - Some PostScript  cartridges  cause  port

timeout errors; this usually happens because they are not finished

processing the current page when sent more data, and they have not

set  the  port busy-flag. Some cartridges redefine the  page  size

regardless of instructions received to the contrary; these devices

print out an A4-size output on two A4-size sheets instead of  one.

Neither of these problems occur on any true PostScript printer  we

have tested or on other cartridges.

4.   KEYWORDS  - The .DATE, .TIME, or .FILENAME keywords are  refreshed

whenever  output--whether  the  output  is  to  the  screen  or  a

peripheral device.  Only the keyword strings are stored, not their

resultant  text.  This means that selecting them for movement  may

create "text placed on text" for the keyword.  This does not occur

for  .FILENAME.  It can occur for .DATE if placed today and  moved

tomorrow,  and occurs often for .TIME.  Zoom Redraw  corrects  the

display.

5.   AUTOSAVE - If an AutoSave hits during a Zoom command, the save  is

canceled.

6.   .DRV  FILES  -  Do not place any non-supplied .DRV  files  in  the

directory with your Tango .DRV files.  Tango software looks at the

.DRV files to present the list of output drivers to the user.   If

non-supplied files exist in the directory, unexpected behavior may

result.

7.   MACROS  -  Because some commands have been changed, and  some  new

commands have been added, you may find that some Macros that  were

recorded  in versions prior to 2.19K may no longer work correctly.

You  will  have to re-record any macros that no longer work  using

version 2.19K.

8.   MACROS  -  If  a  macro  contains two  or  more  separate  editing

commands,  UNDO will only affect the last one; it will not  affect

the entire macro.

GRAPHICS CARDS/TANGO DRIVER LIST

RESOLUTION COLORS   GRAPHICS CARD         Tango DRIVER

640 X 480      IBM MCGA (Monochrome)         MCGA

720 X 348      Hercules Graphics (Mono)      HERCULES

640 X 350  16  ATI EGA WONDER                EGA

640 X 350  16  ATI VGA WONDER                EGA

640 X 350  16  EIZO MD-B07 VGA               EGA

640 X 350  16  EIZO MD-B10 VGA               EGA

640 X 350  16  GENOA SuperVGA                EGA

640 X 350  16  IBM EGA or Compatible         EGA

640 X 350  16  QUAD GTI SuperVGA             EGA

640 X 350  16  Sigma VGA/H                   EGA

640 X 350  16  SOTA  VGA/16                  EGA

640 X 350  16  Tecmar VGA/AD                 EGA

640 X 350  16  Video7 VEGA Deluxe            EGA

640 X 480  16  ATI VGA Wonder                VGA

640 X 480  16  Colorgraphics EGA             EGA

640 X 480  16  EIZO MD-B07 VGA               VGA

640 X 480  16  EIZO MD-B10 VGA               VGA

640 X 480  16  EVEREX Micro Enhancer EGA     EGA

640 X 480  16  GENOA SuperEGA                EGA

640 X 480  16  GENOA SuperVGA                VGA

640 X 480  16  IBM VGA                       VGA

640 X 480  16  IBM PS/2 VGA                  VGA

640 X 480  16  NEC GB1                       VGA

640 X 480  16  Paradise EGA                  EGA

640 X 480  16  PCG VGA                       VGA

640 X 480  16  QUAD EGA ProSync              EGA

640 X 480  16  QUAD GTI SuperVGA             VGA

640 X 480  16  Sigam VGA/H                   VGA

640 X 480  16  SOTA VGA/16                   VGA

640 X 480  16  Tecmar VGA/AD                 VGA

640 X 480  16  Tseng Labs EGA 480            EGA

640 X 480  16  Video7 VEGA Deluxe            VGA;VSEVDLX

640 X 528  16  GENOA SuperEGA                CUSTOM/SVGA

720 X 540  16  Video7 VGA                    VSEV720

800 X 600  16  Ahead VGA Wizard+/Wiz Deluxe  CUSTOM/SVGA

800 X 600  16  AST VGA Plus                  CUSTOM/SVGA

800 X 600  16  ATI ULTRA                     ATI800

800 X 600  16  ATI VGA Wonder                ATI800

800 X 600  16  BOCA Super VGA                CUSTOM/SVGA

800 X 600  16  Cardinal VGA600               CUSTOM/SVGA

800 X 600  16  Chips & Tech. Super VGA       CUSTOM/SVGA

800 X 600  16  Diamond Speedstar             TSENG800

800 X 600  16  Diamond Speedstar 24X         PARA800

800 X 600  16  Diamond Stealth               ORCS3800

800 X 600  16  EIZO MD-B10 VGA               CUSTOM/SVGA

800 X 600  16  EVEREX Micro Enhancer EGA     CUSTOM/SVGA

800 X 600  16  EVEREX ViewPoint 256 / 512    CUSTOM/SVGA

800 X 600  16  GENOA SuperVGA 5400/6400      GEN800

800 X 600  16  GENOA SuperVGA 7400 Series    TSENG800

800 X 600  16  GENOA SuperVGA 8000 Series    ORCS3800

800 X 600  16  Hewlett-Packard VGA           CUSTOM/SVGA

800 X 600  16  OAK OTI-067 VGA               OAK800

800 X 600  16  Orchid Fahrenheit 1280        ORCS3800

800 X 600  16  Orchid ProDesigner Plus       ORC800

800 X 600  16  Orchid ProDesignerII/IIs      ORC800

800 X 600  16  Paradise EGA or VGA           PARA800

800 X 600  16  PCG PHOTON MEGA PLUS VGA      CUSTOM/SVGA

800 X 600  16  QUAD GTI SuperVGA             CUSTOM/SVGA

800 X 600  16  Renaissance RVGA I/II         CUSTOM/SVGA

800 X 600  16  Sigma VGA/H                   CUSTOM/SVGA

800 X 600  16  SOTA VGA/16                   CUSTOM/SVGA

800 X 600  16  STB VGA                       CUSTOM/SVGA

800 X 600  16  STB Powergraph VGA            TSENG800

800 X 600  16  Tatung TVGA-16                CUSTOM/SVGA

800 X 600  16  Tatung VGA                    CUSTOM/SVGA

800 X 600  16  Tecmar VGA/AD                 CUSTOM/SVGA

800 X 600  16  TIGA Hercules Graph Station   TIGA

800 X 600  16  Trident TVGA VGA / 8900       TRI800

800 X 600  16  TSENG 4000                    TSENG800

800 X 600  16  Super VGA                     CUSTOM/SVGA

800 X 600  16  VESA                          VESA800

800 X 600  16  VESA (GRB Color Palette)      VESA8G

800 X 600  16  Video7 VGA                    CUSTOM/SVGA

800 X 600  16  Video7 Fastwrite              VSEV800

800 X 600  16  Wizard Deluxe EGA             CUSTOM/SVGA

800 X 600  16  ZyMOS POACH 51 VGA            CUSTOM/SVGA

1024 X 768 16  ATI VGA WONDER XL             ATI1K

1024 X 768 16  ATI ULTRA                     ATI1K

1024 X 768 16  Diamond Speedstar             TSENG1K

1024 X 768 16  Diamond Stealth               ORCS31K

1024 X 768 16  EIZO B10                      GEN1024

1024 X 768 16  GENOA SuperVGA 5400/6400      GEN1024

1024 X 768 16  GENOA SuperVGA 7400 Series    TSENG1K

1024 X 768 16  GENOA SuperVGA 8000 Series    ORCS31K

1024 X 768 16  OAK OTI-067 VGA               OAK1K

1024 X 768 16  Orchid ProDesigner VGA Plus   ORC1024

1024 X 768 16  Orchid ProDesigner II/IIs     TSENG1K

1024 X 768 16  Orchid Fahrenheit 1280        ORCS31K

1024 X 768 16  Paradise VGA1024              GEN1024

1024 X 768 16  Sigma Designs VGA/X16         GEN1024

1024 X 768 16  SOTA VGA/16                   GEN1024

1024 X 768 16  STB Powergraph                TSENG1K

1024 X 768 16  TIGA Hercules Graph Station   TIGA

1024 X 768 16  Trident 8900 Series           TRI1024

1024 X 768 16  TSENG 4000 Series             TSENG1K

1024 X 768 16  VESA                          VESA1024

1024 X 768 16  VESA (GRB Color Palette)      VESA10G

1024 X 768 16  Video7 VRAM                   VSEV1024

1024 X 768 16  Video7 VRAM II                VSEV1024

1024 X 768 16  Western Digital 90C00         ORC1024

1280 X 1024 16 VESA                          VESA1280

1280 X 1024 16 VESA (GRB Color Palette)      VESA12G