******************* Whats new for GC-PREVUE 6.2   6-21-95 *******************

DRILL RACK AND APERTURE LIST

-  GC-PREVUE now supports 99 drill racks and 99 aperture list positions

instead of the previous one drill rack and 10 aperture list positions.

Instead of the previous "A,B,C,..." aperture list designations, we

now use "a01, a02, a03,...".

Drill racks are similarly referred to as "r01,r02,r03" instead of

the previous "T" designators.

-  The Copy NC function now uses or creates a drill rack based on the

aperture list used in the source layer (or first-used, if multiple

layers are being copied at once).  A01 uses R01, A10 uses R10, and

so forth.

CHANGE IN THE .CWK FORMAT

-  Due to drill rack and aperture list enhancements, old .CWK files can

be read in, but .CWK files written by GC-PREVUE version 6.2+ can't be

read into older GC-PREVUE or other older GraphiCode products.

APERTURE AND CUSTOM APERTURE FILES

-  Aperture lists and custom aperture files written by version 6.2+ can

not be read into older GraphiCode products.  Of course older

aperture lists can be read into 6.2+.

This release also deals with graphics

text problems under certain memory managers (QEMM stealth mode, for

example) and in the OS/2 DOS Window.  Formerly it was necessary to disable QEMM stealth mode, or the text in the graphics viewer was illegible.

Similarly in OS/2 it was necessary to use virtual boot techniques

instead of just running in a DOS window.

******************* What's New for GC-PREVUE version 6.01 ********************

GC-PREVUE is now capable of running in a Windows DOS box.

Certain CAD packages, when writing RS-274-X aperture descriptions for simple

shapes such as round or square, choose to describe them as complex (custom)

apertures consisting of a single round or square.  For pads this is not a

problem, but traces can not be drawn with custom apertures (in GC-PREVUE or

on a photoplotter).  GC-PREVUE converts such apertures to their simplest

form automatically.

What's New for GC-PREVUE version 6.0:

Support for reading RS-274-X and MDA FIRE Autoplot extended Gerber formats.

Support for polygon apertures (filled areas described as their edges instead

of thousands of traces used to sketch them in).

80306 32 bit protected mode code for improved speed and memory usage using

the Rational Systems DOS Extender.

Added checkplot support for Epson and HP Laserjet printers.

More layers (99), more apertures per aperture list (999).

A Physical Layer sense for grouping data layers for positive/negative merges,

and specifying blind and buried vias.

What's New for GC-PREVUE version 5.1:

Version 5.1 provides the capability to view and checkplot (using Postscript)

jobs using composite layers - that is, layers which "undraw" the layers

below them.  This is useful for ground plane isolations.

The key features to note are:

1) the addition of a layer polarity column in the layer list.

2) the "Negative Layers Draw in: (Black/Color)" selection in the Control Draw

(Ctrl D) dialog box in the Viewer.  When set to "Black", this enables the

viewing of composite data layer effects.

3) Automatic file format recognition on input (choose AUTO from the F1 data

load menu (Gerber, HPGL, ..., Auto)).

There are three layer polarity types:

"+" (positive) - normal plotting - plot items are drawn by turning pixels ON.

"c" (composite) - plot items are drawn by turning pixels OFF.

"-" (negative) - layer begins with all pixels ON, draws by turning pixels OFF.

A combination of "+" and "c" is the usual method for groundplane isolations.

The Negative Layers drawing mode selection lets you view "-" and "c" layers

as if they were positive, or to see the effect of the composite data.

Note also that aperture lists can be edited wherever they can be popped up.

PostScript checkplots are automatically tiled (printed across several pages)

as needed.  A special PostScript flavor for Linotronic machines has been

added.  HPGL checkplots have also been cleaned up a little to accomodate

Houston Instruments' HPGL emulation, and DMPL output has been added.

If you don't already have a GC-PREVUE manual you should download GCPREVUE.ZIP

from the GraphiCode BBS at (206) 771-6980.