GC-PREVUE(tm)

The PCB Photoplot Data Viewer for DOS

User's Guide

Copyright (C) GraphiCode(tm), Inc. 1987-95.  All rights reserved.

No part of this manual may be reproduced by any means, mechanical

or electronic, without the written permission of GraphiCode,

Inc., except for the personal use of the licensee.  The

information in this manual is subject to change at any time.

Rev. D1 February, 1994

The following terms used in this manual are the trademarks of the

indicated companies:  MS-DOS, Microsoft Corp.; PC-DOS, IBM, IBM-

PC,XT,AT,PS/2, International Business Machine Corp.; Hercules,

Hercules Computer Technology; Gerber, The Gerber Scientific

Instrument Co.; GC-PREVUE, GraphiCode Inc.; HPGL, HP LaserJet,

Hewlett-Packard; Epson FX, Seiko Corp.; GRAFPLUS, Jewell

Technologies; Postscript, Adobe Systems Inc.; Quest and Emma,

Marconi Quest CAE; Excellon, Excellon Automation; S & M, Sieb &

Meyer, Sieb and Meyer GmbH; Linotronic, Linotype, Inc.; Excellon,

Esterline Corp.; DFX, AutoCAD, Autodesk, Inc.

LICENSING AGREEMENT

Shareware License Statement

GraphiCode, Inc. grants you, the possessor of a GC-PREVUE program

diskette, the right to make copies of this GC-PREVUE diskette,

and to distribute copies of this diskette with the following

restrictions and requirements:

You must distribute ALL the GC-PREVUE files on this diskette at

the same time.  This diskette contains the files MAKEPVU.EXE and

a README file that includes this License Statement.

You may put the GC-PREVUE files on this diskette on a network or

BBS as long as the contents are unchanged.

You may distribute the GC-PREVUE files ONLY in the compressed

form in which you received them on this diskette.  You may NOT

distribute the files that load onto your hard drive as a result

of running the MAKEPVU.EXE decompression program.

You must NOT modify or rename any GC-PREVUE files or add any

files to the diskette you distribute.

You may sell copies of this GC-PREVUE diskette, but only at a

price that does not exceed the cost of reproduction and

distribution.  If you advertise any GC-PREVUE diskettes, you must

identify "GC-PREVUE" as a trademark of GraphiCode, Inc.

You must NOT copy or distribute printed copies of the on-disk GC-

PREVUE manual, or the printed GC-PREVUE manual published by

GraphiCode, or any portion of those printed manuals.

GC-PREVUE is the only GraphiCode product distributed as

"shareware".  You may NOT make copies of other GraphiCode

software.

You don't need to notify GraphiCode when you distribute copies of

GC-PREVUE according to these rules.

For user support call GraphiCode Technical Support at (206) 771-

5600 or call our technical support BBS at (206) 771-6980.  To

order printed copies of the GC-PREVUE user manual, or for more

information about GraphiCode productsproqducts, contact us at (206)-672-

1980.  Our fax number is (206) 672-2705.  Our address is:

Internet email: support@graphicode.com

GraphiCode, Inc.

6608 216th Street SW, Suite 300

Mountlake Terrace, WA  98043 USA

This agreement is governed by the laws of the State of

Washington, U.S.A..

i

Table of Contents

1.

Introducing GC-PREVUE                   1

1.1 What GC-PREVUE Does              1

1.2 GC-PREVUE Features:              2

1.3 About This Manual                2

1.4 Getting Help                     3

1.5 The History of GC-PREVUE         3

2.

Getting Started with GC-PREVUE          4

2.1 System Requirements              4

2.2 Installing GC-PREVUE             4

2.3 Starting the Program             4

2.3.1 If GC-PREVUE Does Not Run   5

2.4 Leaving the Program              5

3.

GC-PREVUE Basics                        7

3.1 Ctrl and Alt                     7

3.2 Using a Mouse                    7

3.2.1 Clicking                    7

3.2.2 Dragging                    7

3.3 Esc and Ctrl Break               8

3.4 Getting Help                     8

3.4.1 The Help Line               8

3.4.2 ClickCenter or Ctrl M for Menu   8

3.5 Using Menus                      9

3.5.1 Horizontal Menus            9

3.5.2 Vertical Menus              9

ii

3.6 Working with Data Input Lists    10

3.6.1 Moving from Field to Field  10

3.6.2 Editing a Data Field        10

3.6.3 Toggles (Limited Choice Data Fields)  11

3.6.4 Units (Ctrl U)              11

3.6.5 Scrolling Large Lists       11

3.6.6 Printing Lists (Alt P)      11

3.7 Special Features for Filename Entries 11

3.7.1 Interactive Directories     12

3.7.2 Default Filename Extensions 12

3.7.3 Hardware Devices as Output Files 12

3.7.4 Backup Files                12

3.8 Perform a DOS Command (Ctrl C)   13

3.9 Record (Alt F10) &

Playback (Ctrl F10) Keystrokes       13

3.10 RAM Use Display                 13

4.

GC-PREVUE Configuration                 15

4.1 Load a Configuration             15

4.2 Save the Configuration           15

4.3 Bed Size                         16

4.4 Reminder Interval                16

4.5 Graphics Display Type            16

4.6 Beep                             17

4.7 Mouse Sensitivity                17

5.

Aperture Lists                          18

5.1 Special Aperture List Features   18

5.1.1 Aperture Position           18

5.1.2 Aperture Code               18

5.1.3 Drill                       18

5.1.4 Rotate +/-90 (R90)          19

5.1.5 Dimensions                  19

5.1.6 Aperture Type               19

5.1.7 Custom Apertures            20

5.1.8 Polygon Apertures           20

iii

5.1.9 Invisible Apertures         21

5.2 Working with Aperture Lists      21

5.2.1 Load/Create an Aperture List (F1)     21

Load an ASCII Aperture List     21

CAD Aperture List Converters    22

5.2.2 Unload an Aperture List (Ctrl F1)     22

5.2.3 Edit an Aperture List (F2)  22

Remove or Adding Apertures      23

Sort Aperture List              23

Jump                            23

5.2.4 Create Custom Apertures     23

Thermal                         24

Rounded Rectangle               24

Diamond                         24

Bullet                          25

Chamfer Rectangle               25

5.2.5 Inspect an Aperture List (F3)    25

5.2.6 Write an Aperture List to Disk (F4)   25

5.2.7 Copy & Resize an Aperture List (F5)   25

6.

The Drill Rack                          28

6.1 Special Drill Rack Features      28

6.1.1 Diameter                    28

6.1.2 Invisible Drills            28

6.2 Working with the Drill Rack      29

6.2.1 Load a Drill Rack           29

Load an ASCII Drill Rack        29

6.2.2 Save the Drill Rack         29

6.2.3 Edit the Drill Rack         29

Add and Remove Drills           29

Make Drills Invisible/Visible   29

Set Units                       29

Jump                            29

6.2.4 Make a New Drill Rack       30

7.

Working with Plotter and NC Data        32

7.1 The Layer List                   32

7.1.1 Status: Empty, Loaded or Edited  32

7.1.2 X-Offset(inch/mm)-Y         32

7.1.3 Aperture List ("Aps")       32

Automatic Generation of

Aperture Lists and Drill Racks  33

7.1.4 Layer Polarity              33

7.1.5 Plated and Unplated Drill Holes  34

iv

7.1.6 Filename                    34

7.2 Loading Layers from Data Files   35

7.2.1 Gerber, NC Drill and NC Rout Files    35

Layers                          35

Error File                      36

Plotter                         36

Whole Digits                    36

Precision                       36

Absolute/Incremental            36

Zero Suppression                36

Units                           36

Coordinate System               36

Arc Style                       37

Flash/Draw Check (Gerber only)  37

Modal Tool Peck (Gerber only)   37

Character Set                   37

Magtape Filter (Gerber only)    37

X and Y Offsets                 37

Stop on M00, M01                38

7.2.2 HPGL Files                  38

Layers                          38

Error File                      38

Plotter                         38

Ignore chord tolerances         38

Edge Adjust                     39

Units Adjust                    39

7.2.3 Quest (Emma) Files          39

Layers                          39

Error File                      39

Units                           39

Character Set                   39

7.2.4 Auto (Automatic Format Detection)     40

7.3 Working with the Layer List      40

7.3.1 Load (F1)                   41

7.3.2 Unload Layers (Ctrl F1)     41

7.3.3 Physical Layers (Alt F1)    41

7.3.4 View Layer(s) (F2)          41

7.3.5 Layer(s) Report (F3)        41

7.3.6 Check Plot (F4)             42

7.3.7 Copy Layers (F5)            43

7.3.8 Copy NC (Ctrl F5)           44

7.3.9 Save All Work (F6)          44

7.3.10 Restore All Work (F7)      45

7.3.11 Reorder Layers (F8)        45

7.3.12 Set Layer Color (F9)       45

7.3.13 Set Display Units (Ctrl U) 45

7.3.14 Jump to Layer...(Ctrl J)   46

7.3.15 Set...(Ctrl S)             46

7.3.16 Print Layer List (Alt P)   46

v

8.

Viewing Data                            47

8.1 The Viewer/Editor Display        47

8.1.1 The Help Line               47

8.1.2 User X,Y                    48

8.1.3 Absolute X,Y                48

8.1.4 Relative X,Y,d              48

8.1.5 Grids: Step, Snap, & Show   48

8.1.6 Inspect                     48

8.1.7 Setup                       49

The Setup Status Explained      49

Change the Setup                49

8.2 Basic Editing Concepts           50

8.2.1 Getting Help                50

8.2.2 Getting Out of the Viewer/Editor 50

8.2.3 Saving Your Work (F6)       50

8.2.4 Preparing to View:

Hide or View                      50

8.2.5 Positive, Negative and

Composite Layers                  50

Plated and Unplated Drill Holes 51

8.2.6 Pointing, Windowing and Marking  51

Pointing                        51

Windowing (DragLeft/Ctrl W)     51

Marking (M)                     51

Mark All (*)                    52

UnMarking (Alt M)               52

8.2.7 Speed-up Keys (1...9)       52

8.3 Commands for Viewing             53

8.3.1 Undo (U)                    53

8.3.2 Setup (S)                   53

8.3.3 Offset (O)                  54

8.3.4 Rotate (R)                  54

8.3.5 Mirror X/Y data (X) (Y)     54

8.5 Zooming and Panning              54

8.5.1 Pan the Display (DragCenter, P, or Ctrl+Arrow Keys)

54

8.5.2 Zoom In (ClickLeft or +)    55

8.5.3 Zoom Out (ClickRight or -)  55

8.5.4 Zoom Extents (Alt =)        55

8.5.5 Unzoom (Alt -)              55

8.6 Drawing                          55

8.6.1 reDraw Now (D)              55

8.6.2 Drawing Mode (Ctrl D)       55

Style                           55

Negative Layers                 56

Slowdown                        56

vi

Zoom Capture                    56

Inverted Y Axis Orientation     56

PolyFill Factor                 56

Shrink Cursor                   57

8.6.3 Label Apertures (L)         57

8.6.4 Wait Between Layers (W)     57

8.6.5 Aspect Ratio (Ctrl A)       57

8.7 Grid                             57

8.7.1 User-Defined Grid (Alt G)   58

8.7.2 Change the Step Grid Size (F1/F2)     58

8.7.3 Change the Snap Grid Size

(Ctrl F1/Ctrl F2)                 58

8.7.4 Snap Cursor to Grid (G)     58

8.7.5 Freehand Mouse (F)          59

8.7.6 Turn Grid Markers On/Off (Ctrl G)     59

8.7.7 Zero the Relative Readout (Z)    59

8.7.8 User Coordinates Zero (Alt Z)    59

8.7.9 Units mm/inch (Ctrl U)      59

8.8 Jump                             59

8.8.1 Jump to Absolute X,Y (J)    59

8.8.2 Jump to Relative X,Y (Alt J)     59

8.8.3 Jump to Nearest Entity (Ctrl N)  59

8.9 Miscellaneous Commands           60

8.9.1 Save (F6)                   60

8.9.2 Query (Q)                   60

8.9.3 Bed Size (Ctrl B)           60

8.9.4 Inspect Aperture Lists and

the Drill Rack (F10)              61

9.

Converting from CAD Output Aperture Lists    62

GraphiCode Aperture List Conversion Utilities Disk    62

10.

APER, the GraphiCode Custom Aperture Generator    65

10.1 How GC-PREVUE Uses Custom

Apertures                         65

10.2 Using APER to Create Custom Apertures  65

10.2.1 APER Basics              65

10.2.2 CAT Files and CAP Files  65

10.3 Starting APER                66

10.3.1 Custom Aperture Text Descriptions  66

Load                            66

Save                            67

Generate                        67

Edit                            67

10.3.2 The CAT Data Header      67

vii

10.3.3 CAT Syntax               68

10.3.4 Differences Between CAT Syntax

and HPGL                        70

10.3.5 Load CAP Files           70

10.4 APER Macro Feature           70

10.4.1 Help                     71

10.4.2 Merge                    71

10.4.3 Macro Commands Supported 71

10.4.4 Custom Aperture Generation Codes

Macro APER Codes                71

OVAL                            72

OCTAGON                         73

ANNULUS                         75

MOIRE                           76

CROSSHAIR                       78

QUAD                            80

DOUBLE QUAD                     81

THERMAL                         83

ROUND RECTANGLE                 86

10.5 View Custom Apertures While You Edit   87

10.5.1 APER Viewing Features    87

10.6 Custom Apertures in Aperture Lists     88

Index                                   89

viii

1: Introducing GC-PREVUE                           1

1.

Introducing GC-PREVUE

1.1 What GC-PREVUE Does

GC-PREVUE is CAD/CAM software designed to help you work more

efficiently with RS-274-D (often called "Gerber") photoplotter

and NC drill CAD output data (often called "Excellon"), with a

particular emphasis on Printed Circuit Board applications.  GC-

PREVUE supports the extended-Gerber "RS274X" and "MDA FIRE

AutoPlot" formats.  GC-PREVUE also supports HPGL and Quest

(Marconi Emma) plotter formats.

GC-PREVUE lets you look at your PCB fabrication CAD output just

as the photoplotter operator does, and lets you make hardcopy

checkplots of your design.  Think of it as a second opinion,

useful when your CAD system shows one thing but produces another

(a more common situtation than you might think - ask any

photoplotter operator).

By verifying your CAD data from the photoplotter's point of view

you will save time, money and headaches due to miscommunication

with your photoplotting service and fabrication facilities.

GC-PREVUE lets you group multiple data layers (i.e. CAD output

files) into single physical layers, and save this information in

a job file (GraphiCode's .CWK format).  This information is

critical when your design includes layers with positive/negative

file merges or blind and buried vias (see section 7.3.3 for

details).

Job files prepared using GC-PREVUE are fully compatible with

GraphiCode's product line, which includes our PCB CAM software

GC-CAM and our sophisticated preproduction tool GC-PLACE.  There

are also several GraphiCode software packages sold under private

labels that use the .CWK format.  Contact your photoplotting

bureau or PCB fabrication house to see if they accept jobs in GC-

PREVUE .CWK format.  Pass along a copy of GC-PREVUE if they don't

already have it.

GC-PREVUE job files contain all layer data, aperture lists

(including custom aperture shapes) and drill information needed

to effectively communicate with your photoplotting vendor and

fabrication house.

GraphiCode also provides a full line of PCB CAM software for

2                             GC-PREVUE User's Guide

fabrication, assembly and test.  Obviously if you can make the

changes using the original CAD system, you should do so.

Sometimes, however, the CAD system itself is the problem.

1.2 GC-PREVUE Features:

Input of all popular PCB-related data formats, including:

Gerber; Extended Gerber (RS-274-X, FIRE); HPGL;

Quest/Emma; Excellon; Sieb & Meyer

Multi-Layer Data Viewing

Up to 99 layers, 12 assignable colors

Up to 10 aperture lists

Up to 999 aperture positions per aperture list

Positive/Negative/Composite data layer polarity

Traces, pads and holes shown at size

True WYSIWYG custom apertures

Support for most popular aperture shapes

Supports rectangular drawing apertures

Windowed zoom

Data Manipulation for Viewing

Shift by window or Layer

Rotate by window or Layer

Intelligent rotation of asymmetrical pads

Mirror (both axes) by window or Layer

Check Plot: Output user-defined scaled graphics to

Epson, HP Laserjet, HPGL, and Postscript

Disk-based Virtual Memory - Unlimited file size!

32 bit Protected Mode software

Instant Access to any DOS program or command

Built-in Directory for easy file loading

GC-PREVUE is designed for ease of use, with an intuitive

graphics-based Viewer and commands that work with either a single

keystroke or from a menu.

1.3 About This Manual

This manual is written to get you working as quickly as possible.

We recommend that you discover GC-PREVUE's power by running the

software while you read this manual.  The sections listed below

are especially recommended to help get you up and running:

Section 3 - Basic GC-PREVUE operation

Sections 5 and 9 - Aperture lists

Section 7 - Working with plotter and NC data

Section 8 - Inspecting data

Feel free to print this manual for your own use.  If you would

like to purchase a printed copy of this manual, contact

GraphiCode or your local GraphiCode representative.

1: Introducing GC-PREVUE                           3

1.4 Getting Help

If you need further help with GC-PREVUE, you can contact

GraphiCode Technical Support at (206) 771-5600 or call our

technical support BBS at (206) 771-6980 (Telebit WorldBlazer high

speed modem - 1200/2400/9600/14.4K Baud - No Parity, Eight Bits).

See VMM.DOC for more information on out-of-memory problems.

1.5 The History of GC-PREVUE

Since its introduction in 1987, GC-PREVUE has been helping people

who work with PCB CAD output data to develop a better

understanding of that data.  By giving the CAD operator and the

photoplotter operator the ability to inspect the data before

plotting, countless pieces of bad film have been avoided.

Beginning with GC-PREVUE Version 5.0, GraphiCode has provideded

GC-PREVUE as "Shareware" as a service to the PCB industry.  This

means that you are encouraged to use the software and pass it to

your friends and co-workers.  Unlike most shareware, we don't ask

for any user fees.  Mostly we just want to be sure that you to

distribute the complete kit - please read the Shareware license

for the very few restrictions that do apply.  GraphiCode does not

want you to pay for the use of GC-PREVUE, but does reserve the

right to sell printed manuals.  GC-PREVUE and the aperture list

converters are the only shareware products provided by

GraphiCode.  The rest of our product line is provided through

commercial channels and is subject to the usual software license

restrictions.

By easing access to CAD output data verification tools,

GraphiCode is making a contribution to the PCB industry by

helping to educate CAD operators in the nature of CAD output data

and its pitfalls.  Understanding the problem is half the battle.

2: Getting Started with GC-PREVUE                  4

2.

Getting Started with GC-PREVUE

2.1 System Requirements

GC-PREVUE requires:

80386/80386SX/80486 PC compatible

MSDOS or PCDOS version 2.0 or higher

Hercules, CGA, EGA, VGA, SuperVGA, or Hi Res (1024x768)

compatible graphics

2 MB RAM

GC-PREVUE is recommended for use with:

80386/486 PC compatible with 4 or more MB RAM

Math coprocessor

Hard disk drive

EGA, VGA, SuperVGA, or Hi Res (1024x768) color graphics

Microsoft-compatible mouse or equivalent (two or more buttons)

If you have a '286 computer, you can use GC-PREVUE version 5.1,

which is available on GraphiCode's Technical Support BBS (206-

771-6980).  If you do not have a modem, contact GraphiCode or

your GraphiCode representative for a program disk.

2.2 Installing GC-PREVUE

GC-PREVUE is provided as a self-extracting ZIP file called

MAKEPVU.EXE.  The fact that you are reading this means you have

probably already installed the software, but if not, just move to

the directory you want GC-PREVUE to be in and type MAKEPVU.  The

various files (including this document) will be uncompressed into

the current working directory.  It is a good idea to change your

AUTOEXEC.BAT file so that the GC-PREVUE directory is

automatically included in the PATH when your computer is turned

on.  Consult your DOS manual for more information.

2.3 Starting the Program

To start GC-PREVUE, just type:

GCPREVUE

If you have certain data files in mind, you can add them to the

command.  Here is an example:

2: Getting Started with GC-PREVUE                  5

GCPREVUE  DEMODAT1.GBR  DEMODAT2.GBR

If you want to use a certain GC-PREVUE configuration (see Section

4) you can specify that too.  For example, to use "MYCONFIG.PED",

type:

GCPREVUE  /MYCONFIG

Figure 2.1: GC-PREVUE's Start Up Screen

2.3.1 If GC-PREVUE Does Not Run

Common problems include insufficient memory due to too many

memory resident programs (TSRs), or incompatibility with off-

brand "clone" PCs.  Try rebooting your system without any TSRs

loaded.  Also, remember that GC-PREVUE requires at least an

80386SX processor.  See section 1.4 for technical support

information.

See VMM.DOC for more information on out-of-memory problems.

2.4 Leaving the Program

In GC-PREVUE you can always get out of whatever you are doing by

pressing the Esc or Ctrl Break key.  In the main menu, either Esc

or the Exit option will get you out of GC-PREVUE and back to DOS.

GC-PREVUE will ask you if you REALLY want to exit the program -

just press Y or Enter.  Y(es) is the default entry, so make sure

you press N(o) if you do not wish to leave GC-PREVUE.

6                             GC-PREVUE User's Guide

3: GC-PREVUE Basics                                7

3.

GC-PREVUE Basics

This section will tell you what little you need to know about the

GC-PREVUE operating environment, in order to get you moving.

While GC-PREVUE works with Hercules monochrome graphics adaptors,

color devices are preferred (they convey more information).  The

CGA display is driven in its highest resolution mode, which is

also black and white.  This manual will make frequent references

to color which users of monochrome and CGA systems may ignore.

3.1 Ctrl and Alt

In GC-PREVUE, some single key commands are actually a combination

of two keys pressed simultaneously.  When you see, for instance,

a reference to Ctrl M, what is meant is the combination of the

Ctrl and M keys pressed at the same time.  Similarly, Alt M

refers to the Alt and M keys being pressed at the same time.  GC-

PREVUE makes no distinction between upper and lower case letters.

3.2 Using a Mouse

The use of a mouse as a pointing device greatly eases the

operation of GC-PREVUE.  In this section we review a few basic

mouse terms and concepts.

3.2.1 Clicking

The act of "clicking" is simply the pressing and releasing one or

more of the mouse buttons without moving the mouse.  On two-

button mice, the third button is emulated by using both button

simultaneously.  There are three types of clicking:

ClickRight - click the right button only

ClickLeft - click the left button only

ClickCenter - click the middle button.  On two button mice,

click the left and right buttons together.

3.2.2 Dragging

The act of dragging is similar to clicking, except that the mouse

is moved around while the buttons are held down.  A drag ends

when the buttons are released.  In GC-PREVUE, dragging is used

only in the graphical editor.  Dragging is used to move things

around the screen, as if one had pressed one's fingers down on a

8                                GC-PREVUE User's Guide

piece of paper and dragged it around a desktop.  There are three

types of drags:

DragLeft - move the mouse while holding down the left button

only.

DragRight - move the mouse while holding down the right button

only.  Ctrl DragRight means move the mouse while holding down

the right button and the Ctrl key.

DragCenter - move the mouse while holding down the middle

button.  On two button mice, hold the left and right buttons

together.

Dragging can also be accomplished by holding down the appropriate

mouse buttons while using the arrow keys.  This can be useful for

precise shifting of data.

3.3 Esc and Ctrl Break

The Esc key always provides a way to leave from whatever GC-

PREVUE action, area or menu you may be in.

GC-PREVUE treats the Ctrl Break and Esc keys similarly, except

that Ctrl Break works even when GC-PREVUE is not waiting for user

input.  This makes it useful for interrupting screen reDraws and

plotter data read and write operations.

GC-PREVUE uses the Ctrl C key as a means of issuing a Command to

DOS at any time (see Section 3.8).  We mention this here because

some other programs use it as another Break key.

3.4 Getting Help

3.4.1 The Help Line

The second line from the top of the GC-PREVUE screen (or the top

line when in the graphical editor) always contains helpful

information concerning the task at hand.  Help text is always

colored yellow.

3.4.2 ClickCenter or Ctrl M for Menu

You can always review your current options by clicking the center

button of the mouse, which causes a pop-up menu of commands to

appear (see Section 3.5 to learn about menus).  If your hands

happen to be on the keyboard, you can press Ctrl M to achieve the

same effect.  You may select an action or ClickCenter/Ctrl M

3: GC-PREVUE Basics                                9

again to make the menu disappear.  Any command that can be

performed from a menu can also be performed with a single

keystroke.  Each entry in the pop-down menu has its single-key

equivalent that is highlighted in yellow on color monitors or in

brightened text on monochrome monitors.

These pop-up menus serve as a teaching guide to single-key

commands.  As you begin to work faster, you will probably find

yourself bypassing the menu system altogether, but it will always

be there as a memory jogger.  The choice between using menus or

keyboard commands is largely a matter of personal taste, so GC-

PREVUE leaves it up to you.

ClickCenter/Ctrl M will always give you a way out of whatever you

are doing, either by featuring an "Exit" option in the pop-up

menu or by simply acting as if you had pressed the Esc key.

3.5 Using Menus

3.5.1 Horizontal Menus

A horizontal menu presents a list of choices across a single line

of the screen.  Use the mouse or left and right arrow keys to

position the cursor at the desired option.  ClickLeft or press

Enter to select the currently highlighted option.  The main GC-

PREVUE menu is an example of a horizontal menu.

At the main menu, the Esc key causes GC-PREVUE to ask you if you

want to quit the program altogether.  ClickCenter/Ctrl M serves

the same purpose as the Esc key here, since all your options are

already presented in the menu at hand.

Note that each menu entry has either one character highlighted in

yellow, or the entry has Ctrl or Alt plus one character both

highlighted in yellow and in parentheses to the right of the

entry.  Press the corresponding key(s) for the same effect as if

you had positioned the cursor on that menu entry then pressed

Enter.

3.5.2 Vertical Menus

Most menus in GC-PREVUE have their entries arrayed vertically.

Vertical menus work much like horizontal menus, except that the

up and down arrow keys are used.  Each entry in a vertical menu

indicates its single-key command equivalent by highlighting it in

yellow.  There are two kinds of vertical menus:

10                               GC-PREVUE User's Guide

"Pop Down" menus, which appear as a result of actions at a

horizontal menu.  Left and right movements take the cursor out

of a pop down menu and on to the next horizontal menu entry.

"Pop Up" menus, which appear as the result of actions other

than those selected from a horizontal menu.  Pop Up menus

differ from Pop Down menus in that once an action is selected

from a Pop Up menu, the menu goes away.  To exit from a Pop Up

menu either select an action, or press Esc or ClickCenter to

escape with no action.

3.6 Working with Data Input Lists

A common sight in GC-PREVUE is a window containing a list of data

fields for you to fill in.  Remember to watch the Help line for

hints concerning the nature of the input required.  When you have

everything the way you want it, press Enter or ClickLeft to

proceed.  As ever, Esc cancels.

3.6.1 Moving from Field to Field

The arrow keys and the mouse both serve to move the cursor from

one data field to the next.  Tab and Shift+Tab or Ctrl+Left and

Ctrl+Right keys move the cursor to a new field left or right.

The Home and End keys move to the extreme left and right

respectively, while PgUp and PgDn move several lines up and down,

respectively.  To get to the top or bottom of a long list, press

Ctrl PgUp or Ctrl PgDn.

3.6.2 Editing a Data Field

GC-PREVUE features a complete built in line editor.  To change an

existing field, you can:

Erase it completely by pressing Ctrl Backspace.

Type over it, or press the Ins key to insert.  The cursor

changes its shape to cover the entire character when in Insert

mode.

Use Del and Backspace to correct typing errors.

Use Left or Right to move within the field.

Use Ctrl Enter to see the effect of changes to the field

without leaving the field.

GC-PREVUE has special features to help with data fields that take

filenames.  These features are described in Section 3.7.

3: GC-PREVUE Basics                                11

3.6.3 Toggles (Limited Choice Data Fields)

Some fields have a limited range of values such as Yes/No or

Inch/mm.  Such fields are called "Toggle" fields and are

indicated by a special cursor that fills one-half of a

character's height.  You can step through a toggle field's values

using the space bar or ClickRight, or press F10 to see the entire

range of choices.  You can also enter a value directly by

pressing the first letter of the value desired.

3.6.4 Units (Ctrl U)

Anytime that you edit a data field that describes a distance or

an offset, you can change between inch and millimeter units by

pressing Ctrl U.

3.6.5 Scrolling Large Lists

Some lists are too long to fit inside a window all at once.  When

this happens, an up or down arrow symbol appears in the left

column of the window to indicate that there are more data to be

seen.  To get to these data, just keep pressing the up/down or

PgUp/PgDn keys or use the mouse to move the cursor against the

top or bottom of the visible list.

3.6.6 Printing Lists (Alt P)

You can always print a list to a disk file or printer by pressing

Alt P, or by selecting "Print" from the ClickCenter pop-up menu.

Send the list directly to the printer by specifying PRN as the

output filename.  GC-PREVUE does not send a form feed.

GC-PREVUE remembers the last-used filename for list printing and

allows you to add subsequent printed lists to that file.  This

provides a convenient way to maintain a written record of your

work.

Alt P prints complete lists of data that are too long to be

listed on the screen of your computer at one time.

3.7 Special Features for Filename Entries

One special and frequently used type of data field is the

filename.  A filename can include a complete drive and path

designation if desired.  GC-PREVUE has two helpful features to

speed your work with files, Interactive Directories and Default

Filename Extensions.

12                               GC-PREVUE User's Guide

3.7.1 Interactive Directories

Whenever GC-PREVUE asks for a filename, you can press F10 to

select from a directory of files.  GC-PREVUE presents the most

likely wildcard selection based on the type of file in question

(for instance, *.PED for GC-PREVUE configuration files, or *.APR

for aperture lists).  You can modify this if desired, then press

Enter to see a list of files matching the wildcard description.

The wildcard description can contain a complete path designation,

referring to directories other than the current working

directory.

For long lists of files, you can move around the list in the

usual up/down manner, or search for a file by typing up to eight

letters of a filename.  The cursor moves to the right along the

current line to indicate the number of search letters used.  The

Backspace key resets the last search character.  Once the cursor

is positioned on the name of the correct file, press Enter or

ClickLeft to accept it.  As always, the Esc key gets you out

without selecting anything.

3.7.2 Default Filename Extensions

When typing a filename directly, you can save a few keystrokes by

letting GC-PREVUE attach the filename extension (that is, the

".ext" part of "filename.ext").  GC-PREVUE assumes the following

filename extensions:

.PED for configuration files

.CWK for Cam WorK files

.APR for aperture list files

.PDF for plotter definition files

.RCK for drill rack files

.CAP for custom aperture files

GC-PREVUE notes the last-used filename extension for plotter and

NC data input files and treats it as the default.  For output

files, GC-PREVUE uses the default extension specified in the

current Plotter Definition File (see Section 9).

3.7.3 Hardware Devices as Output Files

GC-PREVUE recognizes certain output filenames as DOS device

names, including PRN, LPT1, LPT2, COM1 through COM4, and AUX.

This allows direct output from GC-PREVUE to printers and

plotters, although you must use the DOS "MODE" command to set up

communication parameters.

3.7.4 Backup Files

When you specify a filename for output that matches a file that

already exists, GC-PREVUE will ask you if you want to replace (or

3: GC-PREVUE Basics                                13

in some cases append to) the file in question.  If you elect to

replace the file, GC-PREVUE will offer to create a backup file,

which is a copy of the existing file with the last letter of its

filename changed to "$".

3.8 Perform a DOS Command (Ctrl C)

One convenient feature of GC-PREVUE is the ability to execute any

DOS command (subject to memory availability) from any part of GC-

PREVUE, at any time, without leaving the program.  Just press

Ctrl C and GC-PREVUE will ask you for a command to execute.  Once

the command is completed, GC-PREVUE continues operating where you

left off.

GC-PREVUE presents "COMMAND.COM" as the default choice.  This

command creates a new copy of DOS, which allows you to execute

any series of commands (not just one command).  To return to GC-

PREVUE, make sure you are in the directory where GC-PREVUE

resides and type EXIT at the DOS prompt after completing the

commands.

3.9 Record (Alt F10) &

Playback (Ctrl F10) Keystrokes

A simple macro capability lets you record and playback a sequence

of keystrokes.

To teach a macro, press Alt F10.  GC-PREVUE will ask for a

filename (default extension .KEY) for saving all keystrokes and

mouse events until the next time you press Alt F10.  When you

press Alt F10 to stop recording a macro, GC-PREVUE asks if you

want to actually stop recording or just suspend until Enter or

Esc is pressed again in the current window.  This allows for user

input during the macro playback.

To play back a macro, press Ctrl F10.  GC-PREVUE will ask for a

filename (default extension .KEY) of saved keystrokes and mouse

events.  Macros can call other macros (that is, Ctrl F10 works

during an Alt F10 teaching session), with nesting up to five

deep.

3.10 RAM Use Display

In the top right corner of the screen, GC-PREVUE displays how

much RAM it is currently using.  This is most useful when

assessing job progress or system needs.

14                               GC-PREVUE User's Guide

4: The GC-PREVUE Configuration                     15

4.

GC-PREVUE Configuration

GC-PREVUE provides a means of saving its setup information for

later use.  In addition to the items shown in the "Configuration"

area at the bottom of the main GC-PREVUE screen, GC-PREVUE saves

information such as aperture list selections, plotter definition,

drill rack selection, input data format, graphics display type,

and more.  The configuration also keeps track of file locations,

even if each file type is in a different directory.

4.1 Load a Configuration

At start up, GC-PREVUE will automatically load the configuration

found in file DEFAULT.PED.  This is generally the easiest way to

use configuration files - set GC-PREVUE up the way you like it,

then save the configuration to DEFAULT.PED.  An alternative way

to use configuration files is on the DOS command line.  In order

to start GC-PREVUE with the configuration found in

"MYCONFIG.PED", you could type:

GCPREVUE  /MYCONFIG

To load a configuration from inside GC-PREVUE, select the

Configure option on the main GC-PREVUE menu.  This produces a pop

down menu from which you can select "Load Configuration".  GC-

PREVUE will prompt you for a filename.

Aperture lists and the drill rack are automatically loaded as

part of the configuration loading process.

If GC-PREVUE does not find a DEFAULT.PED in the current directory

when it starts, then it will look in the directory where

GCPREVUE.EXE resides.

4.2 Save the Configuration

To save the current GC-PREVUE configuration, select Save

Configuration from the Configure pop down menu.  GC-PREVUE will

prompt you for a filename.  If you want this configuration to be

loaded automatically at the next start of GC-PREVUE, name the

output file DEFAULT.PED.

16                            GC-PREVUE User's Guide

Figure 4.1: Configuration File Input Window

4.3 Bed Size

When in the GC-PREVUE graphical Viewer/Editor, you will notice a

dark grey rectangle (adjust your monitor if you don't see it)

which represents the extents of the useful plotting area.  You

can set these extents using the Bed Size entry of the Configure

pull-down menu.  You can also change this from inside the GC-

PREVUE Viewer/Editor.

4.4 Reminder Interval

To help prevent you from losing work due to power outages and

other mishaps, GC-PREVUE can periodically remind you to save your

work (via the Save All command).  In the Configuration pull-down

menu, select Reminder Interval (I).  Choose the time between

reminders by typing in a specific number of minutes.  If you

would rather not be reminded to save, select "0" minutes.  The

job-save files (.CWK format) can be sent to your photoplot

provider for use with GraphiCode's GC-CAM photoplotter front-end

software, or with your provider's own copy of GC-PREVUE.  This is

a real help when communicating with your photoplot provider.

4.5 Graphics Display Type

Here you can tell GC-PREVUE what kind of graphics display system

you use.  GC-PREVUE supports the most popular graphics display

4: The GC-PREVUE Configuration                     17

types (Hercules, CGA, EGA, VGA, SuperVGA), and can recognize them

automatically when Autosense Graphics is selected.  Of course you

can also select a specific graphics type, which is useful for

"clone" machines which may not behave according to standard,

making identification difficult.  AutoSense will detect Hercules,

CGA, EGA and VGA adapters.  If you have SuperVGA, you must select

it explicitly since there are several competing "standards" for

SuperVGA graphics.  Try VESA SVGA first, since most newer cards

support this.

GC-PREVUE supports SuperVGA adapters in 800x600 16 color mode.

Select Graphics Display Type from the Configure menu to see a

list of supported SuperVGA adapters.  If your adapter's brand

name is not in the list, try VESA, Tseng or Chips & Technologies.

VESA is an industry standard, and Tseng and C&T are not adapter

brands but rather brands of BIOS chips built into many popular

SuperVGA adapters.  If all else fails, use the custom SuperVGA

and set the AX and BX register values as needed (this is quite

technical, although the needed information may appear in the

manual accompanying your adapter).

If GC-PREVUE thinks that the requested and detected display types

do not match, a warning will be issued but the request will be

granted.  EGA displays can safely do CGA graphics, and VGA can do

EGA or CGA graphics, however:

NOTE THAT THERE IS SOME POSSIBILITY OF DAMAGE TO YOUR SYSTEM IF

YOU SELECT A DISPLAY TYPE OTHER THAN THAT WHICH IS ACTUALLY

INSTALLED!

4.6 Beep

The GC-PREVUE warning tone toggles on and off each time Beep is

selected.  This feature is provided simply for the convenience of

those who prefer a quiet work place.

4.7 Mouse Sensitivity

Use this entry to set the responsiveness of your mouse.

ClickRight decreases sensitivity, and ClickLeft increases

sensitivity.  When your mouse moves at an acceptable speed, press

Enter.

5: Aperture Lists                                  18

5.

Aperture Lists

Each layer can be associated with any one of up to 10 aperture

lists via the "Aps" field of the Layer List.  To change any of

the aperture lists themselves, select "Apertures" from the main

menu.  The aperture lists selections are saved as part of the

system configuration, and are loaded automatically when a

configuration is loaded.  GC-PREVUE is also capable of generating

an aperture list from Gerber data (see Section 7.1.3).

Aperture lists are stored in files with an ".APR" filename

extension.

GraphiCode also provides free aperture list converters to convert

from various CAD packages aperture list outputs to the GraphiCode

.APR format (see Section 9).

This section is divided into 2 parts.  The first part (Section

5.1) describes the GC-PREVUE features for working with apertures.

The second part (Section 5.2) explains how you access these

features and put them to work.

5.1 Special Aperture List Features

GC-PREVUE aperture lists contain, as you would expect,

information about each aperture's shape, size and selection code

("D" code).  In addition, GC-PREVUE aperture lists contain

information regarding optional drill/router use, output

transcoding, and data rotation.  In this section we describe each

field of an aperture description.

5.1.1 Aperture Position

Because a GC-PREVUE aperture contains so much more information

than just its "D" code, and because not all plot data is Gerber

data, an aperture in GC-PREVUE is referred to by aperture

position rather than by D code.

5.1.2 Aperture Code

This is the "D" code used in Gerber data to select an aperture.

This information is needed for reading and writing Gerber-format

data.

5.1.3 Drill

Any one of the up to 999 drills/router bits in the drill rack may

be associated with each aperture.  A value of 0 here indicates

that the aperture is not associated with any drill or router

5: Aperture Lists                                  19

information.  The primary use of this field is in conjunction

with the Copy NC function for creating NC data from photoplot

data when no NC data is available.  It is NOT an enforced

relationship - any aperture may be in a pad stack with any drill.

5.1.4 Rotate +/-90 (R90)

Rotating data is not as simple as it may seem at first.  Data

using a round or square pad looks when the same rotated by 90

degrees, but if your data uses apertures such as ovals and

rectangles, which are not symmetrical in rotation, then a

rotation of +/- 90 degrees could result in an erroneous plot.

This is because pad locations rotate but the pads themselves do

not, since the shape and orientation of an aperture is fixed.  In

the case of custom apertures, even a 180 degree rotation can

result in error.

GC-PREVUE helps solve this problem by allowing you to specify a

substitute aperture position to be used in such rotations.  This

is the "R90 code".  A special function (Alt R) in the aperture

list editor helps you set up these codes, and will even create

rotated copies of custom apertures to take you through a full 360

degree rotation in 90 degree increments.

Example: Aperture position 1 is a 50x100 mil rectangle, an

aperture position 3 is a 100x50 mil rectangle.  The R90 code for

aperture position 1 is set to 3, and the R90 code for position 3

is set to 1.  Thus when a pad using aperture position 1 is

rotated by 90 degrees, GC-PREVUE changes its aperture to position

3, resulting in true and proper rotation of the image.  If the

pad is rotated another 90 degrees, it switches back to using

aperture position 1.

5.1.5 Dimensions

For symmetrical shapes, the dimension is read as a diameter.  For

non-symmetric shapes, the first (major) and second (minor)

dimensions are read as X and Y widths respectively.  For shapes

such as Annulus and Thermal, the dimensions represent Inner and

Outer diameters.  The "Target" shape is described by diameter and

line weight.

For custom apertures, the Dimension information is replaced by

the name of the file containing the custom aperture description.

5.1.6 Aperture Type

GC-PREVUE supports the most popular standard pad shapes as well

as custom aperture shapes.  Standard shapes include: round;

octagon; square; rectangle; oblong; annulus; slit; and a stylized

thermal relief and target.  GC-PREVUE also supports custom and

polygon apertures, as described in the next sections.

20                            GC-PREVUE User's Guide

GC-PREVUE supports the drawing of traces with rectangular

apertures.  A "Draw"-type aperture is assumed to be round.  If a

non-"Draw", non-rectangular aperture is used for drawing, it is

assumed for display purposes to be round and of the diameter

given in the major dimension.  Attempting to draw a trace with a

custom aperture will have unpredictable results.

5.1.7 Custom Apertures

Custom apertures are kept in special-format (.CAP) files and can

be added to an aperture list by giving the filename and then

loading it.

Once a custom aperture is loaded in an aperture list, it becomes

a permanent part of the aperture list.  That is, when the

aperture list is saved to an APR file, the custom aperture

description is also saved.  Once the custom aperture is loaded,

you can change the filename to something more descriptive, if you

want to.  Note that this means that if you somehow change a CAP

file on disk, this does not change the aperture lists in which

this file was previously loaded.

A custom aperture consists of any combination of rectangles,

rounds, and draws.  Drawing can be done with round, square, slit

or polygon tools.  GraphiCode provides a free custom aperture

generation software - contact GraphiCode regarding "APER".

In the aperture list editor, you may define Thermal, Rounded

Rectangle, Bullet, Diamond, and Chamfer Rectangle custom

apertures directly, no disk file needed (see section 5.2.4).

A special aperture type "Cstm Thrm" (Custom Thermal) is provided,

which is just a custom aperture which you have tagged as a

thermal.  This is purely informational but may be important down

the line.

5.1.8 Polygon Apertures

Two aperture types are provided to describe fill areas by their

edges.  An External polygon aperture is used to describe the

outer edge of a fill area as a series of connected draws.  If

there is a "knock out" or empty area in the fill area, this is

drawn with an Internal polygon aperture, immediately following

the External polygon data.

Ordering of the trace data describing a polygon is important.

External/Internal polygon effects require that the Internal

polygon traces immediately follow the corresponding External

polygon traces in a layer's data base.

Nested Internal polygons result in alternating fill and clear

areas.

5: Aperture Lists                                  21

5.1.9 Invisible Apertures

Sometimes it is convenient to disable the display and editing of

data associated with certain positions in an aperture list.

Disabled apertures are referred to as "Invisible".  Any aperture

can be made Invisible.  Data associated with Invisible apertures,

like invisible drills, do not display, copy or edit, and they do

not write to output files.  An Invisible aperture is marked by an

asterisk ("*") next to its Type description.  An aperture list

containing invisible apertures has an asterisk next to its name

in the Aperture Lists area of the main GC-PREVUE screen, and any

layers using an aperture list containing Invisible apertures will

have an asterisk next to their entry in the Setup Status area of

the graphical Viewer/Editor screen.

The invisibility attribute of an aperture can be changed from

within the graphical Viewer/Editor, making it a useful analysis

tool (see section 8.9.4).

5.2 Working with Aperture Lists

To begin working with aperture lists, you must select "Apertures"

from the main menu.  Once in the aperture lists area, press Ctrl

M or ClickCenter for a menu of options as described below.  The

chosen action applies to the aperture list currently highlighted

by the cursor.

5.2.1 Load/Create an Aperture List (F1)

To load an aperture list in a .APR file from disk, just type the

name of the file, or press F10 to select one or more filenames

from a directory.  Once the filename is correct, press F1 to load

the file.  If your aperture list is not in the GraphiCode .APR

file format, see section 9 for information on obtaining aperture

list converters.

If the filename is empty, GC-PREVUE will offer to create a new

aperture list.  GC-PREVUE asks how many positions will be on the

new list, then creates it for you and presents it for editing.

For lists of 24 or fewer positions, the default D code numbering

scheme is that which is used by traditional vector photoplotters.

Be sure to fill in all the dimensions and set the Types

correctly.  A more convenient way to create an aperture list is

to generate it from the plotter data (see Section 7.1.3).

Load an ASCII Aperture List

You can load an ASCII aperture list provided it uses the same

format as a GraphiCode ASCII aperture list.  To see an example of

the GraphiCode ASCII format, use the aperture list editor of any

GraphiCode product to print out an aperture list using Alt P.

22                            GC-PREVUE User's Guide

Once formatted, load these lists just like GraphiCode .APR

aperture lists.

CAD Aperture List Converters

You may obtain an Aperture List Converter program disk from

GraphiCode that contains utilities that convert the aperture

lists of many popular PCB CAD programs to GraphiCode's .APR

format.  Please see section 9 for a list of available converters.

5.2.2 Unload an Aperture List (Ctrl F1)

If you are not using a loaded aperture list, you can save some

memory by removing it from the list of aperture lists.  If the

aperture list has been edited, GC-PREVUE will give you a chance

to save it before erasing it.

Figure 5.1: Aperture List Edit Screen

5.2.3 Edit an Aperture List (F2)

F2 pops up a window containing the currently selected aperture

list's aperture information.  This information can be edited.  To

change any numeric value, just position the cursor on the

appropriate field and type in the new value.  You may also edit

an aperture list while you are checking data in the Viewer/Editor

by pressing F10.

Use ClickCenter to see the menu of commands for editing an

aperture list, which include the ability to:

5: Aperture Lists                                  23

Add a position (Ctrl A)

Remove a position (Ctrl R)

Sort the aperture list on ascending D codes (Ctrl S)

Sort the aperture list on D codes in the traditional Gerber

style (D10...D19,D70,D71,D20...D29,D72,D73) (Alt S)

Automatically assign R90 codes (see section 5.1.4) for

asymmetric and custom pads (Alt R)

Make one or more positions visible or "Invisible" (Ctrl I)

Merge an aperture list at the end of this list (F5)

Remove unused aperture position(s) (Alt U)

Set the units for dimensions (Ctrl U)

Jump to a given aperture (Ctrl J)

The Shift key used in conjunction with function keys F1 through

F12 set the aperture Type as indicated by an on-screen help

display.  If your keyboard does not have F11 and F12 function

keys, use Alt F1 and Alt F2 instead. Review Section 5.1 for the

meaning of each field in an aperture list.  Press F10 to pop up a

copy of the current drill rack for inspection.

Remove or Adding Apertures

GC-PREVUE checks to see if an aperture is in use before deletion

via the Ctrl R "Remove" command in the aperture list editor.  It

then asks for confirmation.  If removal is confirmed, any data

using that aperture are deleted also.  When adding or removing

positions in an aperture list or the drill rack, GC-PREVUE

updates the database to reflect the aperture positions.

Multiple custom apertures can be added to an aperture list at the

same time by tagging more than one filename in the F10 pop-up

directory.  New positions will be created in the aperture list as

needed.

Sort Aperture List

Sorting an aperture list does not affect already-loaded data,

since GC-PREVUE automatically adjusts the data for the new

aperture order.

Jump

The Jump (Ctrl J) command in the aperture list editor accepts

either an aperture position number or a D code.  For example, to

jump to position 7, just press Ctrl J and type "7".  To jump to

the aperture using D code 72, just type "D72".

5.2.4 Create Custom Apertures

Certain custom aperture shapes can be created in the aperture

list editor without the use of APER.  The shapes are thermals,

rounded rectangles, bullets and diamonds.

24                            GC-PREVUE User's Guide

In the aperture list editor, change the Type column to reflect

the aperture type being used: Cstm Thrm for thermals, and Custom

for other uses.

In the Dimension column, give a shape description for the new

aperture.  Begin the description with '=' to distinguish it from

a filename (see the format descriptions for each custom aperture

below).  Once the description is typed in, press F1 to create the

custom aperture in memory (no disk file will be created unless

you write it by pressing F4).

GraphiCode-formatted ASCII aperture lists also work with such

descriptions - the macros are processed at load time.

Use the following formats for each of the supported custom

aperture shapes.

Thermal

=TH,<number of gaps>, <O.D.>,<I.D.>,<gap>,<rotation>,<units>

Example: a four-gap thermal with outer diameter 50 mils, inner

diameter 30 mils, a ten mil gap, rotated 45 degrees:

=TH,4,50,30,10,45,MIL

The ,MIL part can be left off if the current aperture list

display units are mils (that is, units are inferred if none are

specified).  The thermal is drawn with a round tool, which

results in less sharp edges than the slit tool used by APER, but

which is less troublesome for many photoplotters.

Rounded Rectangle

=RR,<width>,<height>,<corner radius>,<units>

Example: a 25 by 12 mm rectangle with 3 mm rounded corners:

=RR,25,12,3,MM

Again, if current display units are mm, the units may be omitted.

The rectangle is drawn as four traces surrounding a small

rectangle.

Diamond

=DI,<width>

Example: a .010 inch wide (and .010 inch tall) diamond:

5: Aperture Lists                                  25

=DI,.010,INCH

Again, if current display units are inch, the units may be

omitted.  The diamond is drawn as a diagonal trace using a slit

aperture.

Bullet

=BU,<x size>,<y size>,<direction>,<units>

Example: a 25 by 12 mm bullet pointing right:

=BU,25,12,0,MM

Pointing left:

=BU,25,12,1,MM

A 12 x 25 bullet pointing down:

=BU,12,25,1,MM

Again, if current display units are mm, the units may be omitted.

The bullet is drawn as a rectangle with a round at one end.

Chamfer Rectangle

=CR,<width>,<height>,<chamfer radius>,<units>

Example: a 25 by 12 mm rectangle with 3 mm chamfered corners:

=CR,25,12,3,MM

Again, if current display units are mm, the units may be omitted.

5.2.5 Inspect an Aperture List (F3)

To view an aperture list's contents without changing them,

position the cursor on the list in question then press F3.  You

may also inspect an aperture list while you are checking data on

the view/edit screen by pressing F10.

5.2.6 Write an Aperture List to Disk (F4)

Once you have set up an aperture list to your satisfaction, you

will want to save it for future use.  It's best to save

immediately using this option.  However, GC-PREVUE will make sure

you get a chance to save an edited aperture list before leaving

the program or loading a new aperture list in its place.

5.2.7 Copy & Resize an Aperture List (F5)

Sometimes it is convenient to have a copy of an aperture list

with slightly larger or smaller dimensions for each aperture, for

26                            GC-PREVUE User's Guide

making a silk screen or perhaps a ground plane isolation.  When

copying the currently selected aperture list to the first

available aperture list table slot, GC-PREVUE lets you add or

subtract a given amount from the radius of each copied aperture.

If the aperture list being copied contains custom apertures, GC-

PREVUE will copy and adjust those too.  It is not necessary to

save the new descriptions to disk since they will be included

with the aperture list itself when it is saved to disk.

5: Aperture Lists                                  27

6: The Drill Rack                                  28

6.

The Drill Rack

The Drill Rack is a list of up to 999 drill selection codes and

diameters used when reading, writing, creating and especially

viewing/editing NC Drill data layers.  The diameter information

allows drill locations to be displayed at width.  All layers and

all aperture lists refer to the same single drill rack.

The current drill rack selection is displayed in the

Configuration area at the bottom of the main GC-PREVUE screen.

You can use GC-PREVUE's aperture list editor to associate each

aperture in an aperture list with any one (or none) of the drills

in the drill rack.  This information is used by the Copy NC

function in the GC-PREVUE Layer List to create NC drill data

layers from photoplot data layers, and optionally by the output

function for transcoding.

You may inspect the drill rack while in the aperture list editor,

and you may inspect the drill rack or any aperture list while in

the Viewer/Editor.

6.1 Special Drill Rack Features

6.1.1 Diameter

To change the display diameter of a drill, just position the

cursor on that drill's diameter value and type in a new value.

Press Ctrl U to change units.

6.1.2 Invisible Drills

Sometimes it is convenient to disable the display editing and

output of data associated with certain positions in the drill

rack.  Data which use  invisible drills do not display, edit,

copy or write.  If the drill rack contains Invisible entries, an

asterisk ("*") appears by the drill rack entry in the

Configuration display area of the main GC-PREVUE screen.  In the

graphical Viewer/Editor, an asterisk appears next to the Setup

Status entry for each layer using the drill rack if the drill

rack contains invisible drills.

The Invisibility attribute of a drill can also be changed from

within the graphical data Viewer/Editor, making it a useful

analysis tool.

6: The Drill Rack                                  29

6.2 Working with the Drill Rack

When you select the Drills option from the GC-PREVUE main menu, a

menu appears allowing you to variously create, edit, save and

recall drill racks.  The menu selections are as follows:

6.2.1 Load a Drill Rack

When Load is selected from the Drills menu, GC-PREVUE offers to

save the current drill rack if it has been edited.  GC-PREVUE

then prompts for a filename for loading the next Drill rack.

Load an ASCII Drill Rack

You can load an ASCII drill rack provided it uses the same format

as a GraphiCode ASCII drill rack.  To see an example of the

GraphiCode ASCII format, use the drill rack editor to print out

an drill rack using Alt P.  Once formatted, load these lists just

like GraphiCode .RCK drill racks.

6.2.2 Save the Drill Rack

When Save is selected from the Drills menu, GC-PREVUE prompts for

a filename for saving the current drill rack to disk.

6.2.3 Edit the Drill Rack

When Edit is selected from the Drills menu, GC-PREVUE pops up a

copy of the current drill rack for inspection and editing.  To

change a value, just position the cursor on the value and type in

a new value.  As usual, ClickCenter or Ctrl M present a menu of

commands, as described below:

Add and Remove Drills

GC-PREVUE allows you to change the number of entries in a Drill

Rack.  Ctrl A and Ctrl R Add and Remove drills respectively.  GC-

PREVUE will ask you to verify the action before deleting a drill.

Make Drills Invisible/Visible

Pressing Ctrl I set the Invisibility of one or more drills.

Set Units

Pressing Ctrl U sets the drill rack display units to inch, mm or

mil.

Jump

Ctrl J pops up a window asking which drill to jump to.  This is

useful for large drill racks.

30                            GC-PREVUE User's Guide

6.2.4 Make a New Drill Rack

When New is selected from the Drills menu GC-PREVUE offers to

save the current drill rack if it has been edited, then prompts

for the number of positions in the new rack.  GC-PREVUE then

brings up the new rack for editing.

Drill racks can also be created "on the fly", based on the T

codes found while reading NC drill data files (see Section

7.1.3).  Such racks still require editing to set tool diameters.

6: The Drill Rack                                  31

7: Working with Plotter and NC Data                32

7.

Working with Plotter and NC Data

This section covers the essentials of reading, and manipulating

photoplotter, HPGL, and NC drill/rout data.  Section 8 covers

viewing in greater detail.

7.1 The Layer List

The Layer List takes up most of the main GC-PREVUE screen, and is

where the majority of your work is done.  Up to 99 layers can be

in the list.  To get to the Layer List, select the "Layers" entry

from the main (horizontal) menu.  Note that not all 99 layers fit

on-screen at once - just scroll the list using the up/down keys.

In this section we'll explain each of the columns in a Layer List

entry.

7.1.1 Status: Empty, Loaded or Edited

This field indicates the presence and status of data on a layer.

"Empty" indicates that the layer is unused.  When the status is

"Loaded", data file has been read in to this layer but has not

been changed.  When the status reads "Edited", the layer has been

changed in some manner.

7.1.2 X-Offset(inch/mm)-Y

The "Offset(inch/mm)" entries indicate the offset to be applied

to each layer for display and output purposes.  Such shifting can

be achieved either by typing in the offsets directly at the

appropriate field in the Layer List, or by entering the graphical

area and using Offset or Shift functions.

The shift values are displayed in inch or metric units depending

on the last units used in the editor or while loading data.  The

Ctrl U key changes the display from inch to metric and back

again.

7.1.3 Aperture List ("Aps")

Each layer must be associated with any one of the up to ten

different aperture lists, or with the drill rack if the layer

contains NC drill/rout data.  An aperture list is referred to by

the letter (A through J), which appears next to its name in the

aperture lists table just below the Layer List area of the main

GC-PREVUE screen.  The drill rack is denoted by the letter "T".

7: Working with Plotter and NC Data                33

Automatic Generation of

Aperture Lists and Drill Racks

If the Aps selection for a layer refers to an empty drill rack or

aperture list entry, GC-PREVUE will offer to create an aperture

list or drill rack based on the input data file for that layer.

When the layer is loaded (as described in Section 7.3.1), GC-

PREVUE will confirm that the aperture list or drill rack is

intentionally blank and then will construct a list containing the

aperture or drill selection codes found.  Dimensions will be

assigned a non-zero, yet unusual value for initial viewing

purposes.  This means that you must edit the new aperture list or

drill rack to assign proper dimensions (see Section 5.2.3 for

aperture editing, and Section 6.2.3 for drill rack editing).

Additionally, if GC-PREVUE encounters an unknown tool code while

reading a file using an existing aperture list or drill rack, GC-

PREVUE will offer to append the new code to the aperture list or

drill rack in use.  Again the new aperture or drill will have an

unusual diameter and must be edited to view properly.

7.1.4 Layer Polarity

Many raster devices (including PostScript, LaserJet, and Epson

checkplots) support both positive and negative data polarity.  By

using combinations of positive and negative data layers, many

effects (such as ground plane isolation) that traditionally

required tedious photo-mechanical techniques can now be produced

on first-generation film.

GC-PREVUE can display negative layers in the normal selected

display color, or in black for true WYSIWYG viewing.  Polarity

for each layer is displayed and changeable in the layer list,

just to right of the aperture list selection:

"+" symbol for positive (draw) layers

"-" symbol for negative (inverse video) layers

"c" symbol for composite (undraw) layers

A "Positive" layer assumes that all pixels are initially off, and

turns pixels on to create features.

A "Negative" layer assumes that all pixels are initially on, and

turns pixels off to create features.  When Control Draw (Ctrl D)

has been used to set "Negative Layers Draw In: Black", a Negative

layer first fills the film extents with its assigned layer color,

then draws on top of this in black.

A "Composite" layer makes no assumptions about initial pixel

values, but turns pixels off to create features.  When Control

Draw (Ctrl D) has been used to set "Negative Layers Draw In:

Black", a Composite layer draws in black but, unlike a "Negative"

layer, does not first fill the bed extents.

34                            GC-PREVUE User's Guide

7.1.5 Plated and Unplated Drill Holes

For drill layers, polarity is irrelevant.  Instead of +/-/c

setting, GC-PREVUE has a setting for plated (p) or unplated (u)

drill holes.

7.1.6 Filename

Most layers are associated with a disk file of plotter or NC

data, although data layers can be created in the editor or by

using the Copy and Copy NC functions.  You can change the

filename for a layer at any time, but the filename must be that

of a real file when you go to load a layer from disk (see the

following example).

To enter or change a filename, just position the cursor in the

Filename column and type in a new name.  To enter one or more

filenames selected from a directory, do the following:

Position the cursor in the Filename column of the first layer

which is to receive a filename and press F10.

A wildcard entry window pops up.  The default wildcards show

the last used filename extension (for example, "*.GBR").  You

can change the wildcards to match any set of files in any

directory (consult your DOS manual for more information on

wildcards and paths).

Press Enter to pop up a list of filenames that match the

wildcard selection.

Select files from the list by positioning the cursor on each

desired filename, followed by ClickRight or the Spacebar.

Selected filenames are displayed in inverse video (black on

white).  Cancel file selections by repeating the ClickRight or

Spacebar action.  To find a filename in a long list of

filenames, type in the first few letters of the desired name.

The BackSpace key lets you change the search letters.

ClickLeft or press Enter to copy all the selected filenames to

the Layer List.

EXAMPLE For a job that consists of the 3 layers "GCDEMO1.GBR",

"GCDEMO2.GBR", and "GCDEMO7.NC", begin by positioning the cursor

on the first unused line of the Layer List, then press F10.  Set

the wildcard window to read "GCDEMO?.*", and press Enter.  The

list of matched files pops up.  Select the desired files from the

list and press Enter again.  Since GCDEMO7.NC contains drill

information, the "Aps" aperture list selection for that layer

should be set to "T", meaning that it uses the drill rack

directly.  The layers can now be loaded using F1.

7: Working with Plotter and NC Data                35

7.2 Loading Layers from Data Files

The Load (F1) option from the Layer List menu lets you load

layers from the disk files indicated in the Filename column.  GC-

PREVUE pops up a menu from which you can select one of several

different input data formats, including:

Gerber (RS-274-D, RS274X, FIRE)

NC Drill

NC Rout

HPGL

Quest (Marconi Emma)

AUTO

The use of each of these input formats is explained in the

following sections.  AUTO scans the file before loading and

determines its data format automatically.

GC-PREVUE can generate or add to drill racks and aperture lists

while reading input data files (see Section 7.1.3).  Drill racks

and aperture lists created or expanded in this manner require

editing to set tool diameters for display.

7.2.1 Gerber, NC Drill and NC Rout Files

Gerber (RS-274-D) and NC data files have very similar formats, so

we'll treat them here in a single section.  The main difference

is that Gerber files are read using an aperture list while NC

files are read using the drill rack.

Gerber and NC data files tend to appear in dialect, which is to

say that they adhere to format standards rather loosely.  For

this reason, GC-PREVUE asks you to specify a plotter definition

file (PDF) and lets you change various PDF input parameter

settings before attempting to read a file.  Once the data files

have been read, GC-PREVUE gives you a chance to save any PDF

changes for future use.

GC-PREVUE will load Gerber RS-274-X and MDA FIRE file formats,

which include aperture information.  Use the Gerber or Auto input

format when loading these files.

GC-PREVUE presents an input window containing the fields

described below.  Most of the fields describe aspects of the

input data format.  (If you are unfamiliar with the terms, see

Section 7.1.  Better yet, just use the Auto input option.)

Layers

Any or all layers can be specified for loading.  Note that if

multiple layers are to be loaded at once, they must use identical

36                            GC-PREVUE User's Guide

data formats.  Layers are specified by number.  Acceptable

specifications include:

all

3

1,2,4,6

1-7

not 4,8

Error File

Typical errors while reading a file include misuse of flash/draw

apertures and use of apertures not declared in the aperture list.

To log such errors to a file in addition to displaying them on-

screen, provide a filename here.

Plotter

Enter the name of a previously created plotter definition file

(PDF), or leave this field blank and fill in the following fields

manually.  You can also specify a PDF here then modify it by

changing the following fields.  GC-PREVUE lets you save any

modifications after the data are loaded.

Whole Digits

This field specifies the input format in terms of the number of

digits to be read as whole units.

Precision

This field specifies the input format in terms of the number of

digits to be read as fractional units.

Absolute/Incremental

This field specifies absolute or relative input data.  This

setting may be overridden by codes in the data file.

Zero Suppression

Choose from "Leading," "Trailing" or "None."  Leading zero

suppression is the most common.

Units

This field specifies inch or metric (mm) input data.  This

setting may be overridden by codes in the data file.

Coordinate System

This selects the coordinate system for loading the data.  Choose

Absolute or User Coordinates.  If User Coordinates are selected,

7: Working with Plotter and NC Data                37

data will be loaded using the User Coordinate system origin. (See

section 9.2.5 for more details).

Arc Style

Select either "Quadrant" or "Full 360."  Most photoplotters and

NC machines use only Quadrant arc styles, which allow for

positive-only I and J codes.  You can toggle to Full 360 if the

data are intended for a plotter with Full 360 degree (signed I

and J) arc data recognition.

This entry is useful for NC Drill input files, since drill files

may also contain rout information, and vice versa.

Flash/Draw Check (Gerber only)

GC-PREVUE will warn you of the cross-use of apertures (Flash with

Draw-type aperture, or vice versa) if the Flash/Draw check is set

to "Yes".  This is primarily of use on older vector plotters.

Set Flash/Draw check to "No" if your target plotter is a laser

plotter or some other raster device.

Modal Tool Peck (Gerber only)

On most photoplotters, a flash occurs only when a data block

contains a D03 code.  This is the Modal Tool Peck "No" case, and

is standard for photoplotters.  Some plotters depart from the

standard and flash at each block after a D03 code until a D02 or

D01 comes along, whether each block contains a D03 or not.  This

is the Modal Tool Peck "Yes" case.

Character Set

GC-PREVUE can read and write ASCII, EIA, or EBCDIC data.  ASCII

is by far the most commonly used character set, while EBCDIC is

common in files downloaded from IBM mainframe computers.  Many NC

drill and rout machines read EIA data.

Magtape Filter (Gerber only)

Enabling the Magtape Filter causes GC-PREVUE to ignore carriage

returns, linefeeds, nulls and spaces when reading Gerber data.

This is useful for data files that have been received from

mainframes or other workstations that insert padding characters

and new lines at regular intervals (sometimes in the middle of a

data value).

X and Y Offsets

These fields allow you to read data for devices with fixed

coordinates systems which differ from the selected coordinate

system.  Any X or Y offset given will be added to all coordinates

read.  This feature works independent of the selected coordinate

system.  (See Section 9.2.39 for further details).

38                            GC-PREVUE User's Guide

Stop on M00, M01

If GC-PREVUE encounters M00 or M01 on input, you may choose

whether or not to ignore these optional stop codes.  Selecting

"No" causes GC-PREVUE to ignore them.

7.2.2 HPGL Files

HPGL files can be loaded on layers using either the Drill rack or

an aperture list.  Pen 1 corresponds to position 1 of the

aperture list or drill rack, pen 2 to position 2, and so forth.

The entire HPGL instruction set is implemented, including

polygons.  Polygons are scan-converted to vectors (filled) as

they are loaded.

Layers

Any or all layers can be specified for loading.  Note that if

multiple layers are to be loaded at once, they must use identical

data formats.  Layers are specified by number

Error File

To log any data errors to a file in addition to displaying them

on-screen, provide a filename here.

Plotter

Proper interpretation of HPGL depends heavily on certain scaling

parameters.  Many CAD programs fail to include these in their

output HPGL data, depending instead on the default scaling

parameters for the target plotter.  GC-PREVUE can achieve the

proper scaling if it knows the plotter model and paper size for

which the data were intended.  Press F10 to select from a list of

plotters and paper sizes.  Selections marked "EXPAND" assume that

the EXPAND hardware feature for this plotter is Yes.

If your HPGL data files include IP and SC commands, the Plotter

selection is unimportant since these commands override the

default scaling.

Ignore chord tolerances

GC-PREVUE can load HPGL arc information two different ways: as

true arcs or as connected line segments (vectorized arcs).  When

"Ignore chord tolerances" is "Yes," arcs are stored as true arcs

(endpoints and arc center).  When "Ignore chord tolerances" is

"No", GC-PREVUE converts arcs to vectors based on the HPGL CT

(Chord Tolerance) command, or to a default of one vector per 5

degrees of arc.

7: Working with Plotter and NC Data                39

Edge Adjust

When reading HPGL polygon information, GC-PREVUE can compensate

for tool width so that polygons fill exactly to the programmed

dimensions.  This is done by shrinking the programmed polygon

edges in by one tool radius, which is the Edge Adjust "Yes"

state.  When Edge Adjust is "No", polygons will be filled based

on the programmed centerlines of the polygon edges, which results

in a polygon whose actual dimensions are one tool radius greater

than programmed.

Units Adjust

A surprisingly common error in CAD output data is the assumption

that one HPGL unit is equal to .001 inch.  This is not true!  HP

defines the HPGL unit as follows:

1 HPGL unit         =    0.025 mm (0.0098425 inches)

40 HPGL units  =    1.0 mm

1016 HPGL units     =    1.0 inch

By setting Units Adjust to "Yes", you tell GC-PREVUE to correct

for this scaling error when reading HPGL data.

7.2.3 Quest (Emma) Files

Quest files (or Marconi Emma, as they are sometimes known) can be

loaded on layers using either the Drill rack or an aperture list.

Slide I0 corresponds to position 1 of the aperture list or drill

rack, slide I1 to position 2, and so forth.

Layers

Any or all layers can be specified for loading.  Note that if

multiple layers are to be loaded at once, they must use identical

data formats.  Layers are specified by number.

Error File

To log any data errors to a file, in addition to displaying them

on-screen, provide a filename here.

Units

This field specifies inch or metric (mm) input data.

Character Set

GC-PREVUE can read and write ASCII, EIA or EBCDIC data.  ASCII is

by far the most commonly used character set, while EBCDIC is

common in files downloaded from IBM mainframe computers.  Many NC

drill and rout machines read EIA data.

40                            GC-PREVUE User's Guide

7.2.4 Auto (Automatic Format Detection)

If you don't know a file's data format, GC-PREVUE figures it out

for you.  From the Load command (F1) on the Layer List, just

select Auto.  GC-PREVUE asks for a list of layers to load (they

don't all have to use the same data format) and an optional error

log filename.  It also asks for an optional approximate board

size - set this to 0 if you don't know.  Auto can make a better

judgement on the data format if it knows about how big the board

is meant to be.

If Auto loads incorrectly, reload using an explicit data format

(i.e., Gerber 2,4 incremental).  Incorrect loading can occur if

the data format is unusual, even if it is valid.  GC-PREVUE uses

a heuristic approach to identify the file format that is usually

correct but is not fail safe.  Use discretion!

MDA FIRE Autoplot aperture list headers are recognized on input.

Paint and scratch within special shapes is not supported.

7.3 Working with the Layer List

Most of the action in GC-PREVUE centers on the Layer List area,

including reading, checkplotting, editing and copying data

layers.

Figure 7.1: Layer List Menu

7: Working with Plotter and NC Data                41

7.3.1 Load (F1)

This function provides for loading layers from disk data files of

various formats.  (See Section 7.2 for specific information on

using each of these input formats.)

7.3.2 Unload Layers (Ctrl F1)

Unloading a layer means erasing all data on the layer.  GC-PREVUE

will ask you to confirm this action, since once data is erased it

cannot be recovered except by re-reading a data file, and any

editing is lost permanently unless it has been written (F4) or

saved (F6).  You have the option of keeping or clearing the layer

name as you unload.

7.3.3 Physical Layers (Alt F1)

Group data from several layers into one Physical layer with this

feature.  You may also refer to this Physical layer wherever a

list of layers is requested.  Up to 34 Physical layers may be

specified.  Physical layers are important when a job contains

layer merges or blind and buried vias.  Such information is vital

to the board manufacturer.

For instance, an inner layer (physical layer 2) of a board might

consist of two composite layers, data layers 3 and 4.  Select

"Physical... (Alt F1)" and you can specify Physical layer 02 as

consisting of data layers 3 and 4.  Now when you want to work

with data layers 3 and 4 together, you can just specify "P2"

(Physical 2) instead of "3,4" at the "Layers:" prompt.

Plated and unplated through holes are listed separately in the

Physical layers set up window - as through holes, they are part

of every physical layer and need not be explicitly included in

individual physical layers.

Drill layers which represent blind or buried vias must be

explicitly associated with the necessary physical layers.  For

example, a buried via which connects inner physical layers 4 and

5 must be identified on both the "Physical 04:" and "Physical

05:" lines.

7.3.4 View Layer(s) (F2)

F2 takes you to GC-PREVUE's multi-layer graphical data Viewer,

which is described in detail in Section 8.

7.3.5 Layer(s) Report (F3)

Report (F3) provides reports across multiple layers using

different aperture lists, including mixed drill and photoplot

layers.  The report includes:

42                            GC-PREVUE User's Guide

Layer's data extents

Aperture list (or drill rack) used

Pad and trace counts by aperture

Sample X,Y location for each aperture used

Total pad and trace count

The data extents take the dimensions of apertures and drills into

account.

7.3.6 Check Plot (F4)

This feature enables you to output data to HPGL and DMPL

penplotters, printers that support Postscript, HP LaserJet

printers, and Epson compatible printers.  Press F4 to pop up menu

of Check Plot controls, which are:

7: Working with Plotter and NC Data                43

Output file: Select the output filename (press F10 for a list

of possible names).

Layers: Specify layer(s) that you wish to Check Plot.

Plotter: Press F10 for a complete list of supported printers

and plotters.  Select from Epson compatibles (9 pin or 24 pin),

HP LaserJet, HPGL and DMPL penplotters, and Postscript.  The

Linotronic selection is just Postscript with special setup

codes.

Color: This selection applies only to Postscript and Linotronic

outputs.

DPI: For HP LaserJet output you can select either 150, 300 or

600 dots per inch (DPI).

Style: Pick from the output drawing styles of Normal,

Transparent, and Ball & Stick.

Pen Width: Specifies the pen width for filled penplots and

unfilled Postscript output.

Edge Adjust: Select "Yes" to adjust pen thickness on polygon

fills (reducing fill edges by one pen width).  "No" fills

following the center line instructions.

Chord Angle: Set this to "0", unless you are using an HP

plotter that needs to control the degrees of arc per vector for

the vectorized output of arcs.  For HP plotters with native arc

capability, this value is used only if it is non zero, and then

it is used as part of the AA command - GC-PREVUE doesn't do the

vectorization, it suggests a value to the plotter.

X Y Scaling: Establish the scale of the output relative to the

data.

X Y Offset: Define offsets here that are compatible with the

target plotter.  This feature accommodates plotters where 0,0

is at the middle of the plotter.  These offsets are applied

before scaling.

Pads: Output pads by setting to "Yes".

Traces: Output traces by setting to "Yes".

Page Size, Margins and Overlap:  For Postscript plots larger

than the given page size, GC-PREVUE will produce a tiled

(multi-page) plot.  These fields tell GC-PREVUE how to set up

the tiling.

7.3.7 Copy Layers (F5)

44                            GC-PREVUE User's Guide

"Copy" copies data from one or more layers to a single target

layer.  If the target layer is not empty, GC-PREVUE offers to

clear it first, otherwise the copied data is appended to the

existing data.  The new layer uses the filename of the source

layer, substitutes the filename extension ".CPY", then encloses

the new filename in parenthesis.  Layers are copied with their XY

offsets added in.

A layer cannot be copied to itself.  Mixing layers which use

different aperture lists will have unpredictable results.

7.3.8 Copy NC (Ctrl F5)

The Copy NC function is provided to allow quick and easy

generation of NC drill and router information from photoplot pad

and trace data.

This powerful GC-PREVUE function behaves like Copy (F5) except

that the target layer is automatically set to use the drill rack

(the "Aps" selection is set to "T" to indicate the presence of NC

data).  The aperture selection for each copied pad or trace is

translated to a drill selection, based on the T code translation

values found in the source layer's aperture list.  The new layer

uses the copied layer's filename except that it replaces the

filename's extension with ".NC" and encloses the new filename in

parenthesis.

Pads and traces using aperture positions with a T code

translation value of zero do not copy.

Mixing layers that use different aperture lists is quite

acceptable, since all layers use the same drill rack.

7.3.9 Save All Work (F6)

This feature, which also appears in the graphical Viewer/Editor,

allows you to quickly save the work in progress on all layers to

a single disk file.  It is a good idea to save your work often,

since you never know when power failures or other disasters might

come along.

Press F6 to begin saving.  A window pops up with an entry area

for a filename and the DOS Verify option.  Enter a filename (up

to 8 characters) and GC-PREVUE will attach the extension of "CWK"

for Cam Work File.  A CWK file contains all layer information, in

addition to all loaded aperture lists, the drill rack, and other

values, such as X,Y counts and offsets.  As such, it forms an

excellent archiving tool.

Other GraphiCode products read in .CWK files, including GC-PLACE and our PCB

CAM software, GC-CAM.  There are also several private

label GraphiCode products that use the .CWK format.

7: Working with Plotter and NC Data                45

The Verify option lets you write your .CWK files using the DOS

function Verify, which reads back files to verify that proper

archiving is being done.  This also slows down file writing, but

it is a good idea when your data is important and your disk is

unreliable.  If you have already set Verify while in DOS, this

switch has no effect.

GC-PREVUE versions 5.1 and earlier write .PWK files, which can be

read by all GraphiCode products including GC-PREVUE 6.0.

7.3.10 Restore All Work (F7)

This feature allows you to restore work saved using Save All

(F6).  This can be a lifesaver, but it can't work if you don't

use Save All first!

In addition to loading CWK files, this feature can also load PWK

files, the work files produced by older version of GraphiCode's

GC-PREVUE.

7.3.11 Reorder Layers (F8)

Move one or more layers to any position in the layer list.  To

reorder, you must:

Layers: Enter the layer(s) to be moved.

Above: Specify where the you wish to move the layer(s) to.  If

you want to move to the top of the list, enter "1."  For the

bottom of the list, enter "100."

In the Viewer/Editor, the top (lowest-numbered) layer is the last

to draw and it overwrites any layers that may be below it.  It is

generally convenient to place NC data layers at the top of the

layer list so that drill positions can be viewed when the

Viewer/Editor is in its normal (non-transparent) drawing mode.

You may also move layers inside of the Viewer/Editor.

7.3.12 Set Layer Color (F9)

This option sets the color for displaying a layer, and it allows

you to select different colors for pads and traces on the same

layer.  GC-PREVUE pops up a window that contains two columns of

the available colors.  Select a color by positioning the cursor

on the bar with the desired color and pressing Enter in each

column.  Or, press the indicated key to quick-select a color.

You may also set colors within the Viewer/Editor.  One color is

assigned per layer unless you specify different colors here.

7.3.13 Set Display Units (Ctrl U)

This sets the units for displaying the X and Y layer offsets.

46                            GC-PREVUE User's Guide

7.3.14 Jump to Layer...(Ctrl J)

Move the cursor to any of GC-PREVUE's 99 layers by pressing Ctrl

J and specifying the layer number.

7.3.15 Set...(Ctrl S)

Establish several layer attributes on multiple layers using Set.

Press Ctrl S to bring up a menu that allows you to set:

Layer(s): Enter the layer(s) that use the attributes

Status: Assign the layer status of hide or View or Edit

Image Polarity: Set positive / negative / composite / or Flip

Drill: Change to plated (p) or unplated (u).

Aperture: The aperture list to be used (A - J or T)

Pad color: Select a color for pads

Trc color: Selectl a color for traces

Physical: Specify physical layers in terms of data layers

The entry No Change means that the layer(s) selected will keep

the attributes presently assigned.  "Flip" in the image polarity

setting allows you to step through each polarity.

7.3.16 Print Layer List (Alt P)

In addition to the contents of the Layer List, Alt P prints the

current aperture list and drill rack filename selections.  This

provides a complete record of the current job status.

8: Viewing Data                                    47

8.

Viewing Data

The GC-PREVUE Viewer provides for the graphical manipulation of

pads and traces of data layers.

8.1 The Viewer/Editor Display

The GC-PREVUE Viewer/Editor display consists of two principal

areas: The larger area being the data display area; and the

smaller being the help/status/setup area along the top and right

of the screen.  The data cursor, a white crosshair, stays in the

data area unless it is directed the Setup area.  Place the cursor

in the Setup area by pressing the S key or simply by moving the

data cursor to the far right of the screen.  To re-enter the data

display, press Esc or Enter, or move the cursor over again with

the mouse or arrow keys.

Figure 8.1: GC-PREVUE Viewer/Editor Screen

8.1.1 The Help Line

The top line of the screen displays helpful messages concerning

the state of the Viewer/Editor and hints concerning data entry.

48                            GC-PREVUE User's Guide

8.1.2 User X,Y

While GC-PREVUE maintains all data in terms of its internal

absolute coordinate system, it may be useful for the user to work

in a coordinate system offset from the absolute system.  GC-

PREVUE supports this completely, accepting most inputs in terms

of the user coordinate system instead of absolute coordinates.

The User Coordinate system origin is set by pressing Alt Z in the

Viewer/Editor.

8.1.3 Absolute X,Y

This gives the current data cursor position in absolute plotter

bed coordinates.

8.1.4 Relative X,Y,d

This gives the current data cursor position in coordinates

relative to the last position at which the Relative display was

zeroed (via pop-up menu or the Z key).  Also, GC-PREVUE shows the

linear distance (d) between the current and last-zero cursor

positions.  These values are useful for measuring and precise

shifting.

8.1.5 Grids: Step, Snap, & Show

The GC-PREVUE data cursor moves on a user-defined grid.  The Step

Grid display indicates the current data cursor movement step

size.  Except when moved by the mouse with Freehand Mouse mode

set to "Yes", the data cursor always moves in steps equal to the

size of the Step Grid.

The Snap Grid is very important when manipulating data: if you

wish, GC-PREVUE will help you keep data on the snap grid by

rounding relevant values when shifting, mirroring and so forth.

The Snap grid should be set no finer than the resolution of your

target output device (photoplotter, NC drill, etc.) so that no

output rounding errors will be introduced.

The Snap Grid is aligned to the User Coordinate system.

When grid markers are displayed, they represent the Snap Grid if

Auto Snap is on, otherwise they represent the Step Grid.  When

grid markers are displayed, a third display, "Grid Shown",

indicates the resolution of the displayed markers.  The grid

markers are often spaced 5, 10, or 50 times farther apart than

the actual Grid points, depending on the current zoom scale.  GC-

PREVUE spaces the markers this way to avoid completely flooding

the screen with grid markers.

8.1.6 Inspect

You can inspect a board one area at a time using the Ctrl key

with any of the cursor movement keys (arrows and Home, End, PgUp,

8: Viewing Data                                    49

PgDn).  After each zoom or pan, the helpline indicates how many

screens are needed to cover the board at the current zoom level,

and which screen is currently displayed.

For example, "view: X 2/4  Y 1/3" indicates that there are 4

screens in X and 3 in Y, and that you are looking at the second

of four in X and the first of 3 in Y.  Ctrl UpArrow would cause a

pan up one screen, with a new helpline "view: X 2/4  Y 2/3".

Screen 1/1 would be in the lower left hand corner.

8.1.7 Setup

The Setup Status Explained

The Setup area contains parameters for viewing layers.

Information for each layer, which is both displayed and

changeable here, includes:

Aperture list selection: A - J, and T for drill layer(s)

Layer viewing status: "V" to view, "h" to hide

Positive/Negative/Composite polarity setting: +, -, or c for

aperture.  Or, for drill layers, plated/unplated drill holes

An asterisk after the aperture list selection for a layer shows

that the selection contains Invisible entries, which may affect

the display and editing of data for that layer.

Change the Setup

To modify the Setup area, press the S key, or move the data

cursor to the right until it moves out of the viewer area and

into the Setup area.  The Setup window expands to cover most of

the viewer/editor and shows the layer names.

To change a setup value for any layer, just put the cursor on the

current value and type in the new one.  Also, certain commands

from the Layer list also work here:

Physical (Alt F1): group data layers into physical layers

Report (F3): obtain a report on one or more layers.

Copy (F5): Copy one or more layers to another layer.

Copy NC (Ctrl F5): Create an NC layer from non-NC layers.

Reorder (F8): move one or more layers within the list.

Color (F9): set the display colors for the current layer.

Set... (Ctrl S): set attributes for many layers at once.

Jump (Ctrl J): jump to another layer.

List (F10): pop up a list of highlighted entry (aperture list

etc)

Print (Alt P): print window to disk or printer

Layer names are editable in this list, but layer loading and

unloading are only available in the layer list on the main menu.

50                            GC-PREVUE User's Guide

F10 pops up a copy of the layer's aperture list selection for

inspection.  Inside the Setup area, ClickCenter/Ctrl M presents a

pop up menu of these commands.  ClickLeft, Enter and Esc all

return the cursor to the viewer area.

8.2 Basic Editing Concepts

8.2.1 Getting Help

Use ClickCenter/Ctrl M to see a pop-up menu of commands (see

figure 8.3).  Press H or ? for a few more hints.

8.2.2 Getting Out of the Viewer/Editor

All you have to do is press the Esc key.  Since Esc is also a way

to quit actions within the editor, GC-PREVUE will ask you to

press Esc again if you want to return to the main GC-PREVUE

screen.  If you don't, just press any other key, or move the

mouse.

8.2.3 Saving Your Work (F6)

It's a good idea to save your work frequently, in case you change

your mind about something, or perhaps there's a power loss.  The

F6 works here just as it does in the Layer List area of the main

GC-PREVUE screen, saving all current layer data, aperture lists,

and drill rack.  To reload previously saved work, return to the

Layer List area and use Restore (F7).  The Configure menu off the

main menu provides for a reminder interval to occasionally prompt

you to save your work.  We strongly recommend that you use it.

Also, the resulting .CWK file containing all job information can

be sent to your photoplot provider as a no-questions statement of

what you want the job to look like.

8.2.4 Preparing to View:

Hide or View

Each layer can be hidden (does not display), or viewed

(displays).  The setting for each layer is displayed and may be

changed in the third column of the Setup area and is indicated by

the character "h" or "V".

8.2.5 Positive, Negative and

Composite Layers

Many raster devices support both positive and negative data

polarity.  By using combinations of positive and negative data

layers, many effects (such as ground plane isolation) can be

displayed in GC-PREVUE.

8: Viewing Data                                    51

GC-PREVUE can display negative layers in the normal selected

display color, or in black for true WYSIWYG viewing.  Polarity

for each layer is displayed and changeable in the Setup area:

"+" symbol for positive layers

"-" symbol for negative (inverse video) layers

"c" symbol for composite (draw with black) layers.

A "Positive" layer assumes that all pixels are initially off, and

turns pixels on to create features.

A "Negative" layer assumes that all pixels are initially on, and

turns pixels off to create features.  When Control Draw (Ctrl D)

has been used to set "Negative Layers Draw In: Black", a Negative

layer first fills the film extents with its assigned layer color,

then draws on top of this in black.

A "Composite" layer makes no assumptions about initial pixel

values, but turns pixels off to create features.  When Control

Draw (Ctrl D) has been used to set "Negative Layers Draw In:

Black", a Composite layer draws in black but, unlike a "Negative"

layer, does not first fill the bed extents.

Plated and Unplated Drill Holes

For drill layers, polarity is irrelevant.  Instead of +/-/c

setting, GC-PREVUE has a setting for plated (p) or unplated (u)

drill holes.

8.2.6 Pointing, Windowing and Marking

Pointing

A fundamental concept in viewing is telling GC-PREVUE what it is

exactly that you want to look at preparatory to using the Query

function.  This is done simply by pointing to the item of

interest by positioning the data cursor on it, or by drawing a

window around the item(s) of interest then pointing to that

window.  Use either the mouse or arrow keys to point with the

cursor.

Windowing (DragLeft/Ctrl W)

To window with a mouse, DragLeft so that the mouse draws a box

around the data of interest, ending the DragLeft when the box

encloses the area of interest.  To window with the arrow keys,

press Ctrl W and move the cursor so that it draws a box around

the data you wish to window.  Then press Ctrl W again to

establish the window.

52                            GC-PREVUE User's Guide

A pad will be marked for Query when the marking window or pointer

is within one pad radius of the pad center.

A trace will be marked when it crosses the marking window or

pointer within one trace radius.

When two pads or two traces on the same layer overlap each other,

it is possible to mark one or both.  If marking is performed with

a window, overlapping features will all mark.  When marking with

just the cursor, the first item encountered in the data will be

the only one marked.  This allows you to pick apart overlapping

data.  An overlapping pad and trace will both mark, with or

without a window.

Once you have indicated the data of interest, press Q for Query.

(See section 8.9.2 for more information on Query).

8.2.7 Speed-up Keys (1...9)

The keys 1 through 9 act as multipliers in cursor moves and

zooming.  That is, once the 4 key is pressed the cursor moves

four times as far with a single press of an arrow key, an

unwindowed zoom changes the scale by 8x instead of 2x, and a zoom

out goes up four levels instead of one.  A zoom resets the speed-

up level to 1.

8: Viewing Data                                    53

Figure 8.2: Edit Features Menu

8.3 Commands for Viewing

In this and the following sections, each subsection describes a

command and lists its single-key implementation in parenthesis.

8.3.1 Undo (U)

This extremely useful command allows you to reverse the most

recent action such as Rotate or Offset.

8.3.2 Setup (S)

The Setup area contains parameters for viewing layers.

Information for each layer, which is both displayed and

changeable here, includes:

Aperture list selection: A - J, and T for drill layer(s)

Layer viewing status: "V" to view, "h" to hide

Positive/Negative/Composite polarity setting: +, -, or c for

aperture.  Or, for drill layers, plated/unplated drill holes

An asterisk after the aperture list selection for a layer shows

that the selection contains Invisible entries, which may affect

the display and editing of data for that layer.

54                            GC-PREVUE User's Guide

8.3.3 Offset (O)

This feature allows you to shift marked data by an explicit X,Y

amount.  GC-PREVUE presents an input window asking for a list of

layers to be offset and the X and Y shift values.  Further

shifting can be achieved using Ctrl DragRight or by repeating the

Offset command.

The default X,Y offset values are those found in the "rel"

display.  This allows easy point-to-point data shifting when used

with the Zero Relative command.

8.3.4 Rotate (R)

This rotates any layer by 90, 180 or -90 degrees (GC-PREVUE asks

you for the rotation, as well as for a list of layers to rotate).

The center of rotation is user-defined, the default being the

center of the window, or the current cursor position.  GC-PREVUE

has special features to aid in the rotation of apertures which

are not symmetrical (see Section 5.1.5).

8.3.5 Mirror X/Y data (X) (Y)

Mirroring Y means that the Y values of marked data are reflected

or "flipped" top to bottom about a line parallel to the X axis.

The default reflecting line is centered in the Y range of the

marking window, or the current cursor Y if marked by pointing.

If Auto Snap is On, the default value is rounded to align with

the Snap Grid.  Mirror X behaves similarly except that data flip

left and right instead of top and bottom.  GC-PREVUE asks you for

the mirror axis value as well as a list of layers to be mirrored.

8.5 Zooming and Panning

8.5.1 Pan the Display (DragCenter, P, or Ctrl+Arrow Keys)

To pan the display with a mouse, use DragCenter.  The start-of-

drag position will be panned to the end-of-drag position.  For

keyboard panning, press P and move the cursor with the arrow

keys.  Press P again to end panning.

To pan one screen at a time (when inspecting a board, for

example), use the Ctrl key with any of the cursor movement keys

(arrows, PgUp, PgDn, Home, End).  The screen pans one screen in

the given direction.  After each zoom or pan, the helpline

indicates how many screens are needed to cover the board at the

current zoom level, and which screen is currently displayed.  For

example, "view: X 2/4  Y 1/3" indicates that there are 4 screens

in X and 3 in Y, and that you are looking at the second of four

in X and the first of three in Y.  Ctrl-UpArrow would cause a pan

up one screen, with a new helpline "view: X 2/4  Y 2/3".  The

speed-up keys apply.

8: Viewing Data                                    55

8.5.2 Zoom In (ClickLeft or +)

To zoom in on a selected area, DragLeft to make a window around

that area, then position the cursor inside it and ClickLeft or

press the + key.  Without a window, the zoom is centered on the

cursor with a 2x magnification.  The speed-up keys (1...9) cause

a corresponding 2x(speed-up) zoom when there is no window

involved.

8.5.3 Zoom Out (ClickRight or -)

This takes you to the previous zoom level.  Pressing one of the

speed-up keys (1...9) first causes GC-PREVUE to zoom out by

(speed-up) zoom levels.

8.5.4 Zoom Extents (Alt =)

Use this Zoom feature to zoom to the extents of data.  Simply

press the Alt and = keys to display the data to its extents.

8.5.5 Unzoom (Alt -)

The Alt key pressed together with the - (minus) key on the

typewriter area of the keyboard (not the numeric keypad area)

zooms out to the maximum-scale zoom level.

8.6 Drawing

8.6.1 reDraw Now (D)

This immediately updates the screen with all editing changes.

Screen redraws can be interrupted by pressing Ctrl Break.

8.6.2 Drawing Mode (Ctrl D)

This command presents an input window for changing the drawing

behavior of GC-PREVUE, as described below.

Style

This toggle field has sets the style for drawing pads and traces.

Four styles are available:

56                            GC-PREVUE User's Guide

Normal: Pads and Traces are drawn filled at width.

Transparent: Pads and Traces are drawn unfilled at width.

Ball and Stick: Pads are shown in outline while traces are

shown as centerlines only.  This is the fastest drawing style.

Mix Colors: Like Normal, except that entities on overlapping

layers change color where they overlap each other.

Negative Layers

This sets the color for drawing negative and composite data

layers.  You can choose between black or the standard layer color

scheme.  In "Negative Layers: Black" mode, a negative layer first

fills the film box with its assigned layer color, then draws its

data in black.  A composite layer draws in black but does not

fill the film box.

Slowdown

Slowdown sets a timer for slowing down reDraw, which can

sometimes be useful for data analysis.  The larger the number

entered the slower the redraw speed.  Enter "0" for maximum

redrawing speed.

Zoom Capture

When you zoom in, GC-PREVUE optionally saves the current screen

image prior to zooming by setting Zoom Capture to "Yes".  When

you zoom out again, GC-PREVUE restores the saved screen image.

This generally results in faster operation of GC-PREVUE, but in

some cases it is actually faster to redraw the screen than to

restore it, and in any event it can require a great deal of

memory or disk space.  You can disable the zoom capture feature

if you wish by selecting Zoom Capture "No".

Inverted Y Axis Orientation

Some CAD systems represent data on an "inverted" Y axis.  An

inverted Y axis increases in value as it moves down the screen

away from the X,Y axes intersection.  The value of the standard Y

axis increases as it moves up the screen away from the XY, axes

intersection.  Choose "Yes" if you are more comfortable with the

inverted display style.

PolyFill Factor

Adjust the PolyFill Factor to speed up polygon redraws in Normal

drawing mode.  When set to "0", the fills are solid.  When set at

"9" (the maximum), every ninth scanline is used for the fill,

which speeds up redraws.  A setting of "2" or "3" offers a good

balance between speed and legibility.

8: Viewing Data                                    57

Also, in Normal drawing mode invalid polygon data (single-side

polygons) are indicated by Xs at the offending vertices.

Shrink Cursor

Adjust the size of your data cursor downward by specifying a

percentage here.  0% equals coverage of the entire screen, 50%

would cover about half as much, and so on.

8.6.3 Label Apertures (L)

This feature allows you to label pads and traces on-screen, by

aperture position.  Pads are labeled at the center while traces

are labeled at the midpoint.  GC-PREVUE waits for a keystroke

after labeling each layer.  Any key will cause the next layer to

be labeled, except for F10, which allows you to inspect any

aperture list or the drill rack.

8.6.4 Wait Between Layers (W)

"Wait Between Layers" causes the screen to be redrawn one layer

at a time.  After each layer, GC-PREVUE waits for almost any

keystroke before drawing the next layer.  To prevent the drawing

of another layer, press Esc.  Pressing L at this time labels the

last drawn layer, and pressing F10 displays its aperture list or

drill rack.

8.6.5 Aspect Ratio (Ctrl A)

Each brand of monitor has a slightly different screen height to

width ratio, or aspect ratio.  This can result in distortion of

the displayed image.  GC-PREVUE provides the ability to

interactively correct for this distortion.

When you press Ctrl A, a rectangle appears on the screen.  If

your aspect ratio is properly adjusted, the rectangle should

appear to be a square.  If it is not square, use the up/down

arrow keys to make it square.  Once adjusted, press Enter.

VGA displays are assumed to have 1:1 aspect ratios.

8.7 Grid

GC-PREVUE works with two grids, Step and Snap, which have the

same basic resolution.  The Step Grid determines the fineness of

cursor moves.  The Snap Grid provides a convenient means of

aligning the cursor or an entity to a (usually coarser) design

grid.  Both the Step and Snap grids are taken from a user-defined

grid selection, which is described in the next section.

A Freehand feature is provided that allows gridless moves using

the mouse.

58                            GC-PREVUE User's Guide

8.7.1 User-Defined Grid (Alt G)

GC-PREVUE allows you to pick the grid sizes that best suit your

work.  Press Alt G to pop up the grid selection window.  The

first line G0 contains the smallest cursor movement step desired.

On each subsequent line, Gn specifies a multiple of the previous

line.  The Step Grid readout shows the effective grid size for

the current line as you move up and down in the window.

Example If you desire an 1/8 mil grid, enter ".000125" for G0.

This is the smallest grid step.  On the next line G1, enter 2 for

a grid step of 1/4 mil.  For G2, enter 2 for a grid step of 1/2

mil.  For G3, enter 2 for a 1 mil grid.  Now for G4 you might

enter 5 (5 mil) then for G5, 2 (10 mil), and so on down to G10.

(See Figure 8.8 below.)

Figure 8.3: User-defined Grid Setting

8.7.2 Change the Step Grid Size (F1/F2)

Use the F1 key to increase the Step Grid size (i.e. move from the

Gn grid level to the Gn+1) and the F2 key to decrease it.

8.7.3 Change the Snap Grid Size

(Ctrl F1/Ctrl F2)

Increase the Snap Grid size (i.e., move from the Gn grid level to

the Gn+1) by pressing Ctrl F1, and decrease it by pressing Ctrl

F2.

8.7.4 Snap Cursor to Grid (G)

Press G to position the cursor on the nearest Snap Grid point.

8: Viewing Data                                    59

8.7.5 Freehand Mouse (F)

Sometimes it is convenient to move the data cursor completely

unrestrained by grids.  The F key toggles the use of the mouse

between Freehand (gridless) moves and normal (grid-oriented)

moves.  The arrow keys are always Step grid oriented.

8.7.6 Turn Grid Markers On/Off (Ctrl G)

Press Ctrl G to toggle the grid markers display.  Grid markers

reflect the current Step or Snap Grid (if Auto Snap is on) value,

although they may be placed 5, 10, 50 or more times farther apart

to avoid crowding the screen.

8.7.7 Zero the Relative Readout (Z)

This command causes relative X Y offsets and linear distance d to

be measured from the current data cursor position.

8.7.8 User Coordinates Zero (Alt Z)

The origin for the user coordinate system is expressed in terms

of absolute GC-PREVUE coordinates.  (See section 8.1.2 for a

discussion of the User Coordinate system.)

8.7.9 Units mm/inch (Ctrl U)

Ctrl U toggles the display units from inch to mm.  Since GC-

PREVUE maintains all data in its own internal units (1/1000

micron) this setting has no effect on the actual operation of GC-

PREVUE.

8.8 Jump

8.8.1 Jump to Absolute X,Y (J)

Press J to "dial in" a new cursor X,Y position.  Subsequent Jumps

default to the previous Jump position, forming a convenient way

to move between two areas of the plot.

8.8.2 Jump to Relative X,Y (Alt J)

Jump Relative works just like Jump Absolute, except that pressing

Alt J jumps the cursor to a relative X,Y position.

8.8.3 Jump to Nearest Entity (Ctrl N)

Moves the cursor to the center of the nearest pad or the endpoint

of the nearest trace.  Press Ctrl N to activate.

60                            GC-PREVUE User's Guide

8.9 Miscellaneous Commands

8.9.1 Save (F6)

Save your edited data frequently as you work to prevent losing it

due to failures either machine-related or human.  Also, the saved

data represents the job in a single transfer file, which can

greatly aid communication with your photplot vendor.  (See

section 7.3.8 for a complete description of this feature.)

8.9.2 Query (Q)

Query describes data more specifically than Label, with a pop up

window that displays pointed-to data by layer number, aperture,

and XY location.  Simply mark the data that you are curious about

and press Q.  As you scroll through the list, the data cursor

moves to the item currently indicated by the text cursor.  (See

section 8.2.6 for details on marking).

You can Jump to a specific line in the list by pressing the J

key.  GC-PREVUE will ask which line to jump to.  To exit Query,

press Enter to preserve the current view.  Press Esc to restore

the extents of the original window.

Figure 8.4: Query Feature

8.9.3 Bed Size (Ctrl B)

Set the size of your plotter's bed here.  Press Ctrl B and type

in the bed dimensions.  The Bed size usually corresponds to the

8: Viewing Data                                    61

film size, although you can vary it in any way that is

convenient.  The current bed size is represented by a grey box

on the graphics display.

8.9.4 Inspect Aperture Lists and

the Drill Rack (F10)

You can inspect and edit any aperture list or the drill rack by

pressing F10.  When inspecting an aperture list, press F10 again

to inspect the drill rack.  Edit commands may be revealed by

accessing the pop up menu (Ctrl M).  See section 5.2.3 for a

description of aperture list edit commands or section 6.2.3 for a

drill rack.

While inspecting an aperture list or drill rack you can change

the Invisibility of any aperture or drill using Ctrl I.  Such

changes take effect the next time the screen is redrawn.

Remember that data using Invisible apertures or drills do not

edit or display, or output!.  Layers which use aperture lists

containing Invisible entries are marked with an asterisk in the

Setup Status area.

9: Converting from CAD Output Aperture Lists       62

9.

Converting from CAD Output Aperture Lists

One of the biggest headaches in dealing with Gerber data is the

fact that the Gerber data says which tool to use, and where, but

does not actually describe the tool (is it round? square?  how

large?  complex?  if complex, how is it described?).  This is

less true than it used to be as more CAD systems write extended

Gerber (RS274X, MDA FIRE Autoplot), but in most cases you still

have to use an external aperture list document along with your

Gerber files.  There is no standard for these external file

formats, so GraphiCode provides software tools for converting

them to the GraphiCode .APR format.  These tools are available at

no charge from GraphiCode (on the BBS, or by contacting us).

GraphiCode Aperture List Conversion Utilities Disk

This disk (available at no charge from GraphiCode) contains a

number of converters to take your CAD system's ASCII aperture

list description and convert it to GraphiCode's .APR format for

use with GC-PREVUE, GC-CAM, GC-PLACE or any other GraphiCode

product (including OEM products).

Of particular interest is USER2APR, which uses a configuration

file which you can modify to describe your own aperture list

format for conversion.

For custom apertures, use GraphiCode's custom aperture generation

program, APER (see section 10).  APER is also provided on the

Aperture Utilities Disk.

Currently available converters include:

algr2apr.exe - Allegro

apt2apr.exe  - Mentor Graphics

cadm2apr.exe - CADAM

eed2apr.exe  - EE Designer

esh2apr.exe  - Escher-Grad photoplotter

hiw2apr.exe  - HiWire

orc2apr.exe  - Orcad

pcad2apr.exe - PCAD

plan2apr.exe - Tibor Darvis Planmaster

pro2apr.exe  - Protel Autotrax

rep2apr.exe  - PADS-PCB and PADS-2000

tang2apr.exe - Tango PCB

ulti2apr.exe - UltiBoard

mcad2apr.exe - McCAD

nib2apr.exe  - Vutrax

cad2apr.exe  - CADSTAR

apf2apr.exe  - Consultek

gap2apr.exe  - Cad Solutions

vis2apr.exe  - Racal Redac VISULA

9: Converting from CAD Output Aperture Lists       63

set2apr.exe  - ProCAD

env2apr.exe  - Lavenir

igi2apr.exe  - Infinte Graphics

gscn2apr.exe - GSI scanner systems

orbo2apr.exe - Orbotech

xlon2apr.exe - Excellon .DCD format

zukn2apr.exe - Zuken

user2apr.exe - User-definable input format

Also included are eia2asc.exe and ebcd2asc.exe, which convert

files from EIA and EBCDIC to ASCII so that they can be viewed,

edited and converted on a PC.

Also provided is GENERIC.DOC, which contains C language source

code for producing your own converter if you wish to do so.

USER2APR is a simpler way to achieve this, however.

The following is a reprint from GraphiCode's Tech Support BBS

detailing how to go about obtaining a converter if your CAD

system is not already covered.  There is NO CHARGE for this

service.

REPRINTED FROM GRAPHICODE'S TECH SUPPORT BBS - BULLETIN 4

24 HOUR TECH SUPPORT BBS (206) 771-6980  1200/2400/9600/14.4K N81

How to Get a Converter from Your CAD Aperture Lists to

GraphiCode's .APR Format

Although GC-PREVUE can create aperture lists (.APR files) based

on reading your CAD output files, you still have to type in the

proper aperture shapes and sizes in GC-PREVUE's aperture list

editor.  Boring!  But there is a solution...

PADS-PCB users, for example, can download a tool called REP2APR

(in REP2APR.ZIP) which automatically converts a PADS-PCB .REPort

file to a GraphiCode .APR aperture list file.  If you'd like such

a tool for use with your brand of CAD software, here's all you

have to do:

1)  Create a tiny test board which uses one of each GC-PREVUE

standard aperture shape also supported by your CAD software

(round, rectangle, square, octagon, thermal, target, annulus,

etc).  Please use one of each of everything - NOT just the ones

you normally use yourself - this is for everyone's benefit!

2)  Write the test board out as CAD data (Gerber, probably)

along with an ASCII report file describing the apertures used.

3)  Load the CAD data into GC-PREVUE and set up the aperture

list to match what you were seeing in your CAD system.

4)  Create a GC-PREVUE .CWK job-save file.  ZIP this together

with the ASCII report file, the CAD output data, and a note

telling us how to get in touch with you, then upload it to the

64                            GC-PREVUE User's Guide

GraphiCode BBS.  Leave a note to the SYSOP that you have a

request for a new aperture list format converter.

With this information we can create a conversion utility for

use by you and other users of your CAD software.  There is no

charge for this service - all we ask is that you please take

the time to create a concise but THOROUGH data set.  As

converters are created they will be posted in the Utilities

file area for use bythe general public.

10: APER, the Custom Aperture Generator         65

10.

APER, the GraphiCode Custom Aperture Generator

APER is available on the GraphiCode BBS at (206) 771-6980.  It is

not necessary to have APER to use GC-PREVUE.

10.1 How GC-PREVUE Uses Custom

Apertures

Custom apertures differ from other apertures in that they cannot

be described as a simple shape such as Round, Square or Octagon.

A custom aperture consists of a series of lines, arcs and shapes

in a coordinate system that centers on the current plotter X,Y

position.

The simplest way to create a custom aperture is to design it in

GC-PREVUE then use the Teach Customs function.  Another

convenient way is by using the APER program.  With APER you can

easily describe the shape of a custom aperture in an HPGL-like

language.  APER takes this description and translates it to a

format that GC-PREVUE uses to display the aperture.  GC-PREVUE

can also output the aperture as a series of draws and flashes,

allowing you to use custom apertures on non-programmable

photoplotters.

GC-PREVUE supports the use of custom apertures for pads only -

you can't use them for drawing arcs or traces.

10.2 Using APER to Create Custom Apertures

10.2.1 APER Basics

APER is a simple text editor and translator that reads or creates

human-readable (text) custom aperture description files and

translates them to machine-readable (binary) files for maximum

speed when used by GC-PREVUE.

To create a custom aperture, you must first create a description

of the aperture using APER's HPGL-like aperture description

language.  This description can consist of rectangles, circles,

straight lines, and arcs.  Lines and arcs can be drawn at any

width.

APER operates in graphics mode.  As you edit the aperture

description text in one window, APER shows the resulting custom

aperture shape in another window.

10.2.2 CAT Files and CAP Files

APER deals with two kinds of files: Custom Aperture Text (.CAT)

files and Custom Aperture Description (.CAP) files.

66                            GC-PREVUE User's Guide

CAT files contain human-readable aperture descriptions, and can

be prepared within APER or by using any word processor.  CAT

files can also be largely prepared using any CAD system with HPGL

output, but will require some modification in APER to deal with

scaling and line weights.

CAP files contain machine-readable custom aperture descriptions,

and can be produced only by APER.  APER reads CAT files and uses

that information to produce CAP files.  CAP files can be read

only by GC-PREVUE and related GraphiCode software products.

10.3 Starting APER

APER is a stand-alone program, so you must quit GC-PREVUE or use

Ctrl C to get a DOS command line.  At the DOS command line, type:

APER

APER uses the GC-PREVUE configuration file DEFAULT.PED to

determine your graphics display type.  If this file is missing,

APER will automatically determine the graphics adaptor type.

10.3.1 Custom Aperture Text Descriptions

Once APER is running, select Files from the main menu.  A pop

down menu appears, with the following entries:

Load

Use Load to read in existing aperture description text (CAT)

files.  Once the text is loaded, APER displays the custom

aperture shape and places the cursor in the text editing window.

For a list of CAT files, press F10 when APER prompts you for a

filename.  To return to the Files menu, press Esc.

10: APER, the Custom Aperture Generator         67

Figure A.1: APER Custom Aperture Generation Screen

Save

Use Save to write text files back to disk after editing.

Generate

Use this option to generate a CAP file for use by GC-PREVUE.

Generate cannot work unless a valid CAT file has been loaded

(i.e., its contents are shown both in the text and graphics

windows).

Edit

To continue editing the current CAT text after Save or Generate,

select Edit.  The cursor returns to the text editing window.

10.3.2 The CAT Data Header

The first thing a CAT aperture description must do is declare the

working units (inch, mil, mm or micron) and any optional

mirroring or rotation of the working coordinate system.  This

header takes the form:

<units>[MX][MY][RT<degrees>]:

<units> is one of the following:

MM

MICRON

INCH

MIL

MX, MY and RT are used to mirror and rotate the aperture

description.  This allows the generation of CAP files for several

different applications, such as, emulsion up, down, and data

rotation, with minimal editing to a single CAT file.

Header

MM        unit type

MICRON

INCH

MIL

MX        mirror X

MY        mirror Y

RT        rotate                        coordinates

68                            GC-PREVUE User's Guide

HPGL Commands Supported

LT   line type

PT   pen thickness

PU   pen up

PD   pen down

PA   plot absolute

PR   plot relative

AR   arc relative

AA   arc absolute

RR   rectangle relative

RA   rectangle absolute

CI   circle

Figure A.2: Supported HPGL & CAT Commands

10.3.3 CAT Syntax

Other than in the header just described, APER CAT files use HPGL

to describe custom apertures.  The following is a list of

supported instructions and their arguments.  The BOLDFACE

UPPERCASE letters are the supported instructions.  <Brackets>

indicate required arguments, and [braces] indicate optional

arguments.  (See Figure A.2 for a list of all supported

commands).

LT (Line Type)

Format: LT[draw style]

To select the Line Type of an aperture, choose either LT0 or LT1.

LT0 draws with a round aperture, which results in rounded or

half-circle end points.  LT1 draws with GraphiCode's "Slit"

aperture, which leaves flat or squared end points.  LT0 is the

default.

PT (Pen Thickness)

Format: PT<thickness>

Set line thickness for lines and arcs.  Stays in effect until

next PT instruction.

PU (Pen Up)

Format: PU[x,y,[x,y,[...]]]

Following drawing commands update the pen position but do not

actually draw anything.  Optional x,y movement arguments are

10: APER, the Custom Aperture Generator         69

interpreted as absolute or relative based on last seen PA/PR

instruction (default is PA).

PD (Pen Down)

Format: PD[x,y,[,x,y,[...]]]

Following drawing commands result in actual images.  Optional x,y

line draw arguments are interpreted as absolute or relative based

on last seen PA/PR instruction (default is PA).

PA (Plot Absolute)

Format: PA[xa,ya[,xa,ya[...]]]

X,Y pairs for following PU and PD instructions (and optional

movement arguments) are to be interpreted as absolute locations.

PR (Plot Relative)

Format: PR[xr,yr[,xr,yr[...]]]

X,Y pairs for following PU and PD instructions (and optional

movement arguments) are to be interpreted as relative offset

locations.

AR (Arc Relative)

Format: AR<xr,yr,sweep>

Draw an arc of "sweep" degrees from the current pen x,y around

center x+xr,y+yr.  Negative sweep angles result in clockwise

arcs.  Pen position moves to the end of the resulting arc.  PA/PR

state is not affected.

AA (Arc Absolute)

Format: AA<xa,ya,sweep>

Draw an arc of "sweep" degrees from the current pen x,y around

center xa,ya.  Negative sweep angles result in clockwise arcs.

Pen position moves to the end of the resulting arc.  PA/PR state

is not affected.

RR (Rectangle Relative)

Format: RR<xr,yr>

Draw a filled rectangle having one corner at the current pen

position x,y and the opposite corner at the relative location

xr,yr.  Pen position becomes x+xr,y+yr.  PA/PR state is not

affected.

RA (Rectangle Absolute)

Format: RA<xa,ya>

Draw a filled rectangle having one corner at the current pen

position x,y and the opposite corner at the absolute location

xa,ya.  Pen position becomes xa,ya.  PA/PR state is not affected.

70                            GC-PREVUE User's Guide

CI (Circle)

Format: CI<radius>

Draw a filled circle with the given radius, centered at the

current pen position.  Pen position is unchanged.

10.3.4 Differences Between CAT Syntax

and HPGL

Instead of the usual HPGL scaling instructions (IP, SC, etc.) an

APER CAT file must begin with one of the following key words to

define units:

INCH

MIL

MM

MICRON

The MX (mirror X), MY (mirror Y) and RT (rotate coordinates)

commands are not HPGL commands and can appear only as part of the

data header following the units declaration.

The PT (Pen Thickness) instruction uses the same units as other

CAT instructions.  In HPGL, PT units are always mm.  The PT

instruction affects only line and arc drawing.  Rectangles

(RR,RA) and rounds (CI) do not use the PT information since they

are not arcs or lines.

The RR and RA instructions always create a filled rectangle, and

leave the logical pen at the far corner  of the rectangle.  In

HPGL, they create an outline unless commanded to fill, and they

do not update the pen position.

The CI instruction always creates a filled circle.  In HPGL, CI

creates an outline unless commanded to fill.

10.3.5 Load CAP Files

Convert a CAP file into a CAT file for editing with this feature.

Load CAP makes it easy to edit CAP files created in Teach Customs

or for custom apertures without available corresponding CAT

files.

10.4 APER Macro Feature

APER gives you the power to create a sophisticated custom

aperture with a Macro Command.  APER's Macro Command comes with

on-line help, merge capacity, and pre-defined aperture shapes,

which require only minor size adjustments for true custom shapes.

Below is a summary of the Macro feature:

10: APER, the Custom Aperture Generator         71

Help: Describes APER formats

Merge: Combines APER custom aperture files

Macro APER codes: Creates popular custom apertures with user-

defined dimensions quickly

10.4.1 Help

Help describes formats that APER accepts for creating custom

aperture text (CAT) files.  Help lists an example CAT file, data

header commands, HPGL-like commands, and APER Macro Commands.

10.4.2 Merge

Merge allows for CAT files to be appended to current APER editor

data.  When a file is merged, the header will be stripped off the

file being read in.  If no data currently exists in the editor,

the header will remain as part of the aperture description.

Remember, merge does not negotiate differences between headers.

If headers are not alike, APER will create an aperture different

from the one in the file being merged.

Another feature that Merge allows is the power merge feature.  To

perform a power merge, press F10  and a list of CAT files will

appear.  Move the cursor to the files that are to be merged.

Press Spacebar to highlight each file.  When all selections are

made, press Esc, and the new CAT data should appear at the

editor.

10.4.3 Macro Commands Supported

ANNULUS (circular and square)

CROSSHAIR

OCTAGON

OVAL

MOIRE (circular and square)

QUAD

DOUBLE QUAD

THERMAL (circular and square)

ROUND RECTANGLE

The following section describes how to use these Macro Commands.

10.4.4 Custom Aperture Generation Codes

Macro APER Codes

Along with the HPGL supported commands are APER specific codes.

These codes allow the user to easily specify common apertures

types without having to know the corresponding HPGL data.  The

aperture designs are general in nature and allow for complete

user specification.  There are 12 supported symbols, which are

listed above.

A feature that is allowed by APER is to enter minimum formats to

create the APER symbol.  Part of this feature is the ability to

72                            GC-PREVUE User's Guide

input partial APER code descriptions.  For instance, instead of

entering CROSSHAIR, CR or anything in between represents a

crosshair.  The minimum formats will be described in more detail

below.

Parameters declared along with the APER syntax range from 0 to

169.092414 inches.  If the parameters are larger than the limits,

typically, APER will not draw anything, or in some cases it will

draw incorrectly.  APER will treat such values in a relatively

friendly manner, but it seems extreme to require a 14 foot long

aperture for a PCB board.

Another restriction APER has is that apertures over 12 inches in

diameter will not be fully displayed inside the graphical window

extents.

Negative values are not used by APER and will be made positive by

stripping the sign and using the magnitude.  (This does not apply

to angles, where any angle will be used.)

If parameters do not correspond to specified values, APER will

try to create the aperture in the most logical manner.  For

instance, if the inside diameter is greater than the outside

diameter, APER will swap the values assuming that you just

entered the data incorrectly.  Also, if APER cannot use the data,

it will use default values instead.  Default values are discussed

individually for each aperture.  However, some parameters do not

have default values, and must accompany the APER code.

The following describes each APER macro:

OVAL

Format: OVAL<major,minor>

Minimum format: OV<major,minor>

To create an oval, type the key word: OV (or OVAL for more

descriptive APER code).  Then enter the major and minor values,

such as:

OVAL50,30;

10: APER, the Custom Aperture Generator         73

Figure A.3: Oval

OVAL will draw a filled oval with the major and minor axes

centered at the origin.  The major parameter represents the

diameter of the oval along the x-axis. Likewise, minor describes

the diameter along the y-axis.

There are no default values for OVAL, therefore both major and

minor parameters must be indicated to generate an oval of any

kind.  If an incorrect entry is made, an error message stating

"Syntax Error" will be displayed.

OCTAGON

Format: OCTAGON<diameter,ratio>

Minimum Format: OCT<diameter>

To create an octagon, use the key word:

OCT (or OCTAGON for more descriptive APER code)

Then enter the diameter and ratio, such as:

74                            GC-PREVUE User's Guide

OCTAGON50,.414;

Figure A.4: Octagon

The OCT function will draw a filled octagon centered at the

origin.

Diameter determines the diameter along the x and y axis.  There

is no default value for diameter.  If it is unspecified, a

"Syntax error" message will be displayed.

Ratio is used to determine the lengths of sides.  The

calculations are as follows:

Lengths[top or bottom or left or right] = diameter * ratio

Lengths[diagonal] =

sqrt(((diameter - length [top or bottom or left or

right])/2)2*2)

The value for ratio must be between 0 and 1.  If a value outside

of 0 and 1 is given, ratio will default to .414, which will

produce an octagon with equal length sides.

The OCTAGON function allows for minimal inputs to describe the

octagon.  When a default value corresponds with the octagon being

generated, that value does not need to be entered.  The following

case defines rules for using minimal inputs:

10: APER, the Custom Aperture Generator         75

If only one input is given, it will be used by diameter and

ratio will default to 0.414.  One input is the minimum number

required to generate an octagon.  If at least one value is not

entered the "Syntax Error" message will be displayed.

ANNULUS

Format: ANNULUS<outside diameter, inside diameter, shape>

Minimum Format: AN<outside diameter>

To create an annulus, use the keyword:

AN (or use ANNULUS for more descriptive APER code)

Then enter the outside diameter, inside diameter and shape of the

annulus, such as:

ANNULUS50,40,0;

Figure A.5: Annulus

76                            GC-PREVUE User's Guide

The AN function draws a circular or square annulus centered

around the origin.

Outside diameter determines the outer diameter along the x and y

axes.  There is no default value for outside diameter.  If it is

unspecified a "Syntax error" message will be displayed.

Inside diameter indicates the diameter of the inner unfilled area

of an annulus.  It is also used to determine the pen width the

annulus is drawn with.  The mathematical expression for the pen

width is as follows:

pen width = (outside diameter-inside diameter)/2.

The default for inside diameter is 0.  If APER uses the default

for inside diameter, the annulus will appear as a filled circle.

Shape specifies whether the annulus is circular or square.  A 1

indicates that the annulus is square.  0 represents a circular

annulus.  If an invalid value is entered for shape, the annulus

will default to circular.

If the inside diameter is set to a larger number than the outside

diameter, the outside diameter will be set to the larger number.

The ANNULUS function allows for minimal inputs to describe the

annulus.  When a default value corresponds with the annulus being

generated, that value does not need to be entered.  The following

cases define rules for using minimal inputs:

If only two inputs are given, shape will default to 0 or

circular.

If one input is specified, MOIRE will use the defaults for

inside diameter, and shape.  The first value is used for the

outside diameter.  One input is the minimum number required to

generate an annulus .  If at least one value is not entered the

"Syntax Error" message will be displayed.

MOIRE

Format: MOIRE<outside diameter, inside diameter, thickness,

distance, shape>

Minimum Format: MO<outside diameter, thickness, distance>.

To create a moire, use the key word:

MO (or use MOIRE for more descriptive APER code)

Then type the outside diameter, inside diameter, thickness of a

ring, the distance between each ring, and the shape of the moire,

such as:

10: APER, the Custom Aperture Generator         77

MOIRE50,0,5,5,0;

Figure A.6: Moire

The MO function draws a circular or square moire centered around

the origin.

The outside diameter is the outer diameter of the outer ring.

There is no default value for outside diameter.  If it is

unspecified a "Syntax error" message will be displayed.

Inner diameter determines the inside diameter of the innermost

ring.  The default for inside diameter is 0.  If the minimum

format for MOIRE is used, the default value will be used.

The thickness parameter determines the line weight each ring is

drawn with.  Aper will draw a figure with a lineweight of zero

(one pixel width), but be warned, ZERO LINEWEIGHTS MAY NOT SHOW

UP ON PHOTOPLOTTING machines.  There is no default value for the

thickness.  If it is unspecified in the MOIRE format, a "Syntax

error" message will be displayed.

Distance is the space between each ring.  There is no default

value for distance.  If distance is unspecified then the "Syntax

error" message will be displayed.

Shape specifies whether the moire is circular or square.  A 1

indicates that the moire is square.  0 represents a circular

78                            GC-PREVUE User's Guide

moire.  If an invalid value is entered for shape, it will default

to circular.

If any parameter is larger than the outside diameter, the outside

diameter will use that value, and substitute the smaller value to

the other parameter.

The MOIRE function allows for minimal inputs to describe the

moire.  When a default value corresponds with the moire being

generated, that value does not need to be entered.  The following

cases define rules for using minimal inputs:

If only four inputs are given, MOIRE will use the default for

shape, which is 0 or circular.

If three inputs are specified, MOIRE will use the defaults for

inside diameter, and shape.  This means that the first value is

used for the outside diameter, thickness is set to the second,

and distance uses the third value.  Three inputs is the minimum

number required to generate a MOIRE.  If less than three are

entered the "Syntax Error" message will be displayed.

CROSSHAIR

Format: CROSSHAIR<diameter, thickness>

Minimum Format: CH<diameter>

To create a crosshair use the key word:

CR (or use CROSSHAIR for more descriptive Aper code)

Then type the outside diameter, inside diameter, thickness of a

ring, the distance between each ring, and the shape of the

crosshair, such as:

CROSSHAIR50,.5;

10: APER, the Custom Aperture Generator         79

Figure A.7: Crosshair

The CH function draws a crosshair centered around the origin.

Diameter is the length of the line along the x-axis.  There is no

default value for diameter.  If it is unspecified a "Syntax

error" message will be displayed.

Thickness determines the line width of each line in the

crosshair.  The default value for thickness is 0.  APER will draw

a crosshair with a line width of zero (one pixel width), but be

warned, ZERO LINEWEIGHTS MAY NOT SHOW UP ON PHOTOPLOTTING

machines.

If thickness is larger than the outside diameter, the outside

diameter will take that value, and substitute the smaller value

to thickness.

The CROSSHAIR function allows for minimal inputs to describe the

crosshair.  When a default value corresponds with the crosshair

being generated, that value does not need to be entered.  The

following case defines rules for using minimal inputs:

80                            GC-PREVUE User's Guide

If only one input is given, CROSSHAIR will use the default for

thickness, which is 0.

QUAD

Format: QUAD<diameter, gap, angle of rotation>

Minimum Format: QD<diameter>

To create a quad use the key word:

QD (or use QUAD for more descriptive APER code)

Then type the diameter, the gap, and the angle of rotation, such

as:

QUAD50,0,0;

10: APER, the Custom Aperture Generator         81

Figure A.8: Quad

The QD function draws a quad centered around the origin.

The diameter is the diameter of the quad.  There is no default

value for diameter.  If it is unspecified a "Syntax error"

message will be displayed.

Gap is the space between each section of the quad.  If gap is

specified it is guaranteed that all entities of the quad will be

the gap distance from other entities.  The default value for gap

is 0.

QUAD allows the user to specify any amount of angular rotation.

Positive and negative angles are valid.  The default value for

the angle of rotation is 0.  If an invalid value is entered for

angle, it will be set to 0.

If any parameter is larger than the diameter, Quad will set

diameter equal to that value, and will substitute the smaller

value to the other parameter.

The QUAD function allows for minimal inputs to describe the quad.

When a default value corresponds with the quad being generated,

that value does not need to be entered.  The following cases

define rules for using minimal inputs:

If only two inputs are given, QUAD will use the default for

angle, which is 0.

If one input is specified, QUAD will use the defaults for gap,

and angle.  The first value will be used for the diameter.  One

input is the minimum number required to generate a quad.  If no

inputs are entered the "Syntax Error" message will be

displayed.

DOUBLE QUAD

Format: DQUAD<diameter, thickness, gap, angle of rotation>

Minimum Format: DQ<diameter, thickness>

To create a double quad, use the key word:

DQ (or use DQUAD for more descriptive APER code)

Then type the diameter, thickness, gap, and angle of rotation,

such as:

82                            GC-PREVUE User's Guide

DQUAD50,5,0,0;

Figure A.9: Double Quad

The DQ function draws a double quad centered around the origin.

The diameter is the diameter of the whole double quad(not the

diameter of the inside quad).  There is no default value for

diameter.  If it is unspecified a "Syntax error" message will be

displayed.

Thickness determines the width of the two outside quarter rings.

There is no default value for thickness.  If it is unspecified a

"Syntax Error" message will be displayed.

Gap is the space between each section of the quad.  If gap is

specified it is guaranteed that all entities of the double quad

will be spaced gap distance from other entities.  The default

value for gap is 0.

DQUAD allows the user to specify any amount of angular rotation.

Positive and negative angles are valid.  The default value for

the angle of rotation is 0.  If no value is entered for angle, it

will be set to 0.

10: APER, the Custom Aperture Generator         83

If any parameter is larger than the diameter, DQUAD will set

diameter equal to that value, and will substitute the smaller

value to the other parameter.

The DQUAD function allows for minimal inputs to describe the

double quad.  When a default value corresponds with the double

quad being generated, that value does not need to be entered.

The following cases define rules for using minimal inputs:

If only three inputs are given, DQUAD will use the default for

angle, which is 0 and set the other parameters to their

respective inputs.

If two inputs are specified, DQUAD will use the defaults for

gap, and angle of rotation.  The other values will be assign to

their respective parameters.  Two inputs is the minimum number

of parameters required to generate a double quad.  If less

inputs are entered the "Syntax Error" message will be

displayed.

THERMAL

Format: THERMAL<outside diameter, inside diameter, gap, number of

spokes, shape, rotation, angle>

Minimum Format: TH<outside diameter, insides diameter>

To create a thermal, use the key word:

TH (or use THERMAL for more descriptive APER code)

Then type the outside diameter, inside diameter, gap, number of

spokes, shape, rotation, and angle, such as:

THERMAL60,45,15,2,0,45;

84                            GC-PREVUE User's Guide

Figure A.10: Round Thermal

THERMAL60,45,15,4,1,45;

Figure A.11: Square Thermal

10: APER, the Custom Aperture Generator         85

The TH function draws a square or circular thermal centered

around the origin.

Outside diameter is the outside diameter of the outer ring.

There is no default value for diameter.  If it is unspecified a

"Syntax error" message will be displayed.

Inside diameter specifies the inside diameter of the outer ring.

Inside diameter is used along with outside diameter to determine

the line width for drawing the outer ring.  The mathematical

equation for the width is:

width = (outside diameter - inside diameter)/2.0;

There is no default value for inside diameter.  If it is

unspecified a "Syntax Error" message will be displayed.

Gap is the space between each section of the quad.  If gap is

specified it is guaranteed that all entities of the double quad

will be spaced gap distance from other entities.  The default

value for gap is 0.

Number of spokes parameter represents the number of gaps that

will be available in the thermal.  This value can be set to any

number using the circular thermal.  The square thermal limits the

number of gaps to 1 to 4,6 and 8.  If other values are entered

square THERMAL will adjust the number to the closest legal value.

The default value for the number of gaps is 0.

Shape determines whether thermal is circular or square.  A 0

input corresponds to a circular thermal.  1 makes the thermal

square.  If an invalid value for shape is entered, shape will

default to 0 or circular.

Rotation specifies the angle at which a gap center will be

placed.  For example, the thermal in Figure 9 has a gap centered

at 45 degrees.  This is the default value for rotation.  If this

is the angle of rotation that is needed the value does not need

to be entered.

Angle is used for the circular thermal only.  This parameter is a

limit angle that places boundaries on the arc angle of each

spoke.  For example,  if angle is set to 90 degrees, and a 2 gap

thermal was declared the spokes would not be the full length.

Plus the gap would be larger than the specified gap distance.

This feature was added to allow for more flexibility.  Angle must

be a positive value.  If it is negative, the sign will be

stripped.  The default value for angle is 360.0 degrees.  This

value allows for a spoke to be fully drawn with no limits.

THERMAL does the best it can to correct possible input errors.

It will check for the following possibilities and perform logical

steps.

86                            GC-PREVUE User's Guide

If the inside diameter is larger than the outside diameter,

THERMAL will swap diameter values.

If the number of sides makes it impossible to draw the thermal

with the specified gap, the thermal will be drawn with the

maximum number of spokes that it can without corrupting the gap

distance.

If the gap is larger than the inside diameter, the inside

diameter will be set equal to the gap.

The THERMAL function allows for minimal inputs to describe the

thermal.  When a default value corresponds with the thermal, that

value does not need to be entered.  The following cases define

rules for using minimal inputs:

If seven inputs are entered, then the inputs will correspond to

their respective parameters.

If six inputs are given, the inputs will correspond to their

respective parameters but angle will default to 360 degrees.

For a square thermal, angle is not used and therefore does not

need to be entered.

If five inputs are specified, the inputs will correspond to

their respective parameters, but the default values for

rotation and angle will be used.

If four inputs are specified, the inputs will correspond to

their respective parameters, but the default values for shape,

rotation, and angle will be used.

If three inputs are specified, the inputs will correspond to

their respective parameters, but the default values for number

of spokes, shape, rotation and angle will be used.

If two inputs are specified, the inputs will correspond to

their respective parameters, but the default values for gap,

number of spokes, shape, rotation, and angle will be used.  Two

inputs is the minimum description for thermal.  If less entries

are input, the "Syntax error" message will be displayed.

ROUND RECTANGLE

Format: ROUNDRECT<x diameter,y diameter,corner radius>

ROUNDRECT creates a filled rectangle with each corner rounded to

the same specified radius.  To create a rectangle with rounded

corners, type ROUNDRECT.  Then enter the x diameter, the y

diameter, and the corner radius, such as:

10: APER, the Custom Aperture Generator         87

ROUNDRECT 10,2,3;

Figure A.12: Round Rectangle

10.5 View Custom Apertures While You Edit

When APER first reads a CAT file, it graphically displays the

contents (unless the CAT file contains an invalid aperture

description).  As you edit an aperture text description, pressing

Ctrl Enter causes APER to re-evaluate the description text and

re-display the aperture shape.

10.5.1 APER Viewing Features

To inspect the aperture shape more thoroughly, press Enter to

move the cursor into the graphics window.  Now you can access GC-

PREVUE-style viewing features (press ClickCenter/Ctrl M to pop up

menu).

The APER viewer is a subset of the GC-PREVUE graphical

Viewer/Editor.  The features included are: Window; Pan; Speed Up

Keys; Zoom In/Out; Unzoom; Query; Step/Snap Grid; Freehand mouse;

Jump Absolute/Relative; and Zero Relative.

Query operates differently in APER.  Position the cursor on an

item of interest and press Q.  The cursor returns to the text

editing area, pointing to the text that describes the item of

interest.

88                            GC-PREVUE User's Guide

In addition to the familiar relative XY display, APER provides a

third relative indicator L, which stands for Linear.  This

readout indicates the straight line distance between the current

cursor position and the last position where the Zero relative key

was pressed.

Note that the displayed image takes into account any mirroring or

rotation arguments in the CAT data header.  Editing is done only

by changing the description text - press Esc to return to the

text window.

10.6 Custom Apertures in Aperture Lists

To use a custom aperture in GC-PREVUE, it must be part of an

aperture list.  To include a custom aperture in an aperture list,

use the APER Generate function to save the custom aperture as a

CAP file.  Note the filename, exit APER, and return to GC-PREVUE.

Use the GC-PREVUE aperture list editor (see Section 5.2.3) to set

any aperture position's type to Custom.  Then, type the CAP

filename in the Dimensions column for that aperture position.

GC-PREVUE will automatically load the description contained in

the CAP file.  In the future, whenever this aperture list is

loaded the CAP file will be loaded too.

Index                                              89

Index

*  21, 28

.CWK  1

.PWK  45

=  24

AA (Arc Absolute)  69

Absolute X,Y  48

Alt  7

ANNULUS  75

Aperture list  61

Aperture List Converter  22

Aperture lists  15

Aperture position  18

Aperture Type  19

Apertures  18

APR  12

AR (Arc Relative)  69

Arc Style  37

ASCII aperture lists  24

Aspect ratio  57

Asterisk  21, 28, 49, 53

Auto  40

Auto Snap  54

Automatic Format Detection  40

Autoplot aperture list  40

Autosense Graphics  17

AUX  12

Backup file  13

Ball and Stick  56

Bed Size  16, 60

Break  8

Bullet  25

CAP  12

CAT Data Header  67

CAT Syntax  68

CGA  7, 52

Chamfer Rectangle  25

Character set  37

Check Plot (F4)  42

CI (Circle)  70

Click  7

ClickCenter  7

ClickLeft  7

ClickRight  7

Color  45

COM1  12

COMMAND.COM  13

Composite data layers  56

Composite layer  51

Copy & Resize an Aperture List (F5)  25

90                            GC-PREVUE User's Guide

Create an aperture list or drill rack  33

CROSSHAIR  78

Cstm Thrm  20

Ctrl  7

Ctrl C  8

Ctrl DragRight  8

Cursor  11, 57

Custom aperture  65

Custom Aperture Description (.CAP) files  65

Custom Aperture Text (.CAT) files  65

Custom Apertures  20, 23

Custom Apertures in Aperture Lists  88

Custom Thermal  20

CWK  12, 44, 45

DEFAULT.PED  15

Diamond  24

Directory  12

Display the aperture shape  87

DMPL  42

Does Not Run  5

DOS  8, 13

Drag  7

DragCenter  8

DragLeft  8

DragRight  8

Drill rack  15, 18, 28, 61

Edge Adjust  39

Edit an aperture  87

Editing an aperture list  22

Epson  42

Erase  10

EXPAND  38

Extents  55

External polygon  20

F11 and F12  23

Filename extension  12

FIRE  40

Flash/Draw check  37

Freehand  59

Gerber (RS-274-D)  35

Graphics display types  16

Grid  48

Grid markers  48

Grid Shown  48

Ground plane isolation  26

Hercules  7, 52

HP LaserJet  42

HPGL  42

HPGL files  38

HPGL output  66

HPGL scaling instructions  70

Ignore chord tolerances  38

Inch  11

Inspect  48

Index                                              91

Inspecting a board  54

Internal polygon  20

Inverted Y Axis  56

Invisibility  61

Invisible  21

Invisible drills  28

Job files  1

Jump  59

KEY  13

Keystrokes  13

Label pads and traces  57

Laser plotter  37

LaserJet  43

Layer attributes  46

Layers  32

LICENSING AGREEMENT  2

Linear  88

Linotronic  43

Load an aperture list  21

LPT1  12

LT (Line Type)  68

Macro  13

Marking (M)  51

Marking window  52

Millimeter  11

Mirroring  54

Mix Colors  56

Modal Tool Peck  37

MOIRE  76

Mouse  7

Mouse Sensitivity  17

NC drill and router  44

Nearest  59

Negative layer  51

Negative Layers  56

Normal  56

OCTAGON  73

Offset(inch/mm)  32

OVAL  72

PA (Plot Absolute)  69

Pan  49, 54

PCB CAD programs  22

PD (Pen Down)  69

PDF  12

PED  12

Penplotters  42

Physical layer  41

Plated and Unplated Drill Holes  51

Playback  13

Point-to-point data shifting  54

Pointing  51

PolyFill  56

Polygon Apertures  20

Polygons  57

92                            GC-PREVUE User's Guide

Pop-up menu  8

Positive and negative data polarity  33, 50

Positive/Negative/Composite polarity  49, 53

Postscript  42

PR (Plot Relative)  69

Print  11

PRN  12

PT (Pen Thickness)  68

PU (Pen Up)  68

PWK  45

QUAD  80

Query  60, 87

Quest files  39

R90 code  19

R90 codes  23

RA (Rectangle Absolute)  69

Raster  37

RCK  12

Record  13

Relative X,Y,d  48

Reminder Interval  16, 50

Removing positions in an aperture list  23

Restore work  45

Rotation  19, 54

Rounded Rectangle  24

ROUNDRECT  86

RR (Rectangle Relative)  69

Save an edited aperture list  25

Save your work  44, 50

Selectively unmarked  52

Set  46

Setup  47

Shareware License  2

Silk screen  26

Slowdown  56

Snap Grid  48, 54

Snap Grid Size  58

Sort the aperture list  23

Step Grid  48

Step Grid Size  58

T code  44

Thermal  24, 83

Through holes  41

Tiling  43

Toggle  11

Transparent  56

Unmark  52

User X,Y  48

User-Defined Grid (Alt G)  58

Verify  45

VESA  17

Vias  41

View X 2/4  Y 1/3  49

View Custom Apertures While You Edit  87

Index                                              93

View Layer(s)  41

Wait Between Layers  57

Wildcard selection  12

Window  51

Zoom  49

Zoom Capture  56

Zoom in  55

Zoom out  55